注射用重组A型肉毒毒素原液纯化阶段清洁验证方案

Cleaning Validation Protocol for the Purification Stage of Recombinant Botulinum Toxin A for Injection Bulk Drug Substance

方案批准Approval of Protocol:

在下表中签名意味着各部门代表已对本方案所含信息的正确性进行了检查并批准可以用于执行。

The signatures of the representatives of the departments specified down indicate that they had examined this protocol for the correctness of contained data and approved for executing.

| 姓名 Name | 公司/职务 Company/Title | 签名 Signature | 日期 Date |
| --- | --- | --- | --- |
| 编制人 Prepared by |  |  |  |
|  | 迈本/验证工程师 MIRABLE/Validation Engineer |  |  |
| 审核人 Reviewed by |  |  |  |
|  | 生产部 Production Department |  |  |
|  | 生产部 Production Department |  |  |
|  | 质量控制部 QC |  |  |
|  | 质量保证部 QA |  |  |
|  | 质量保证部 QA |  |  |
| 批准人 Approved by |  |  |  |
|  | 生产管理负责人 Head of Production Management |  |  |
|  | 质量管理负责人 Head of Quality Management |  |  |

版本修订索引Revision History:

| 版本 Rev. No. | 修订原因 Revision Reasons |
| --- | --- |
| 01.00 | 新制订 Initial issuance |

目录

Table of Contents

1	缩略语Abbreviations	6

2	目的Purpose	8

3	范围Scope	9

4	描述Description	10

4.1	纯化生产Purification Production	10

4.2	设备、器具及相关清洁程序Equipment, Utensils and Related Cleaning Procedures	13

5	职责Responsibilities	15

6	参考文件References	16

6.1	法规和指南Regulations and Guidelines	16

6.2	文件Documents	17

7	验证指导Validation Guidelines	20

7.1	测试项结构Structure of the Tests	20

7.2	执行说明Execution Instructions	20

7.3	验证报告Validation Report	26

7.4	接受标准Acceptance Criteria	27

8	验证策略Validation Strategy	34

8.1	验证范围Validation Scope	34

8.2	取样器具及取样量Sampling Instruments  and Number of Samples	37

8.3	擦拭取样位置评估Swab Sampling location Evaluation	38

8.4	脏污保留时间和清洁保留时间Dirty Hold Time and Cleaning Hold Time	39

8.5	样品编号原则Principles for Sample Numbering	43

8.6	分析方法和取样回收率Analysis Method and Sampling Recovery Factor	44

9	注意事项Precautions	45

10	测试项列表Tests List	49

11	先决条件确认Prerequisite Verification	53

11.1	签名&培训确认Signature &Training Verification	53

11.2	文件签批确认Document Approval Verification	55

11.3	设备确认文件确认Equipment Qualification Document Verification	58

11.4	公用系统确认文件确认Utilities Qualification Document Verification	63

11.5	分析方法验证文件确认Analysis Methods Validation Document Verification	66

11.6	测试仪器仪表校准确认Test Instruments Calibration Verification	68

11.7	测试仪器仪表确认文件确认Test Instruments Qualification Document Verification	70

12	清洁验证测试项目Test Items of Cleaning Validation	72

12.1	清洁操作及目视检查确认Cleaning Operation and Visual Inspection Verification	72

12.2	DHT淋洗取样确认DHT Rinse Sampling Verification	84

12.3	DHT擦拭取样TOC确认DHT Swab Sampling TOC Verification	93

12.4	DHT擦拭取样微生物限度确认DHT Swab Sampling Microbial Limit Verification	97

12.5	CHT目视检查确认CHT Visual Inspection Verification	101

12.6	CHT淋洗取样确认CHT Rinse Sampling Verification	108

12.7	CHT擦拭取样确认CHT Swab Sampling Verification	112

13	偏差清单Deviation List	116

14	变更清单Change List	117

15	附件清单Attachment List	118

附录Annex 1：擦拭取样位置评估Swab Sampling locations Evaluation	119

附录Annex 2：擦拭取样位置图例Illustration of Swabbing Sampling locations	125

附录Annex 3：样品编号表Sample Number Table	127

- 缩略语Abbreviations

本方案中使用的缩略语如下:

The abbreviations that are used in this protocol are listed in the table below:

| 缩略语 Abbreviation | 定义 Definition |
| --- | --- |
| ADE | 每日可接受暴露量 Acceptable Daily Exposure |
| API | 活性药物成分 Active Pharmaceutical Ingredient |
| BDS | 原液 Bulk Drug Substance |
| CIP | 在线清洗 Clean in Place |
| COP | 离线清洗 Clean out Place |
| CHT | 清洁保留时间 Clean Hold Times |
| CPP | 关键工艺参数 Critical Process Parameters |
| CQA | 关键质量属性 Critical Quality Attribute |
| CV | 清洁验证 Cleaning Validation |
| CVMP | 清洁验证主计划 Cleaning Validation Master Plan |
| DHT | 脏设备保留时间 Dirty Hold Time |
| DOE | 试验设计 Design of Experience |
| DP | 成品 Drug Product |
| DV | 稀释体积 Dilution Volume |
| FDA | 美国食品药品监督管理局 Food and Drug Administration |
| FEMA | 失效模式与影响分析 Failure Mode Effects Analysis |
| GMP | 药品生产质量管理规范 Good Manufacturing Practice |
| HBEL | 基于健康的暴露限值 Health-Based Exposure Limits |
| HVAC | 空调净化系统 Heating, Ventilation and Air Conditioning |
| ISPE | 国际制药工程协会 International Society of Pharmaceutical Engineering |
| MACO | 最大允许转移量 Maximum Allowable Carryover |
| N/A | 不适用 Not Applicable |
| NMPA | 国家药品监督管理局 National Medical Products Administration |
| OOS | 超标结果 Out-Of-Specification |
| OOT | 超标趋势 Out-Of-Trend |
| PDA | 注射剂协会 Parenteral Drug Association |
| PDE | 每日允许暴露量 Permitted Daily Exposure |
| PW | 纯化水 Purified Water |
| QA | 质量保证 Quality Assurance |
| QC | 质量控制 Quality Control |
| RF | 回收率 Recovery Factor |
| RSD | 相对标准偏差 Relative Standard Deviation |
| SIP | 在线灭菌 Steam in Place |
| SMP | 标准管理程序 Standard Management Procedure |
| SOP | 标准操作规程 Standard Operating Procedure |
| SSA | 样品表面积 Sample Surface Area |
| TOC | 总有机碳 Total Organic Carbon |
| TRC | 检测结果修正值 Test Result Correction |
| VMP | 验证总计划 Validation Master Plan |
| WHO | 世界卫生组织 World Health Organization |
| WFI | 注射用水 Water for Injection |
| YY001 | 注射用重组A型肉毒毒素 Recombinant Botulinum Toxin A for Injection |

- 目的Purpose

本清洁验证方案的目的是为了验证和记录与纯化车间内与YY001原液纯化阶段产品接触和非产品接触设备、器具的标准清洁程序能够保证残留物或潜在的微生物（含细菌内毒素）在预先所定义的可接受范围内，以防止出现能够对下一批生产产品的安全性和质量带来不利影响的污染。

The purpose of this cleaning validation protocol is to verify and record that the standard cleaning procedures for the equipment and utensils in the purification workshop that product contact and non-product contact with the bulk drug substance of YY001 during the purification stage can ensure that the residues or potential microorganisms (including bacterial endotoxin) are within the predefined acceptable range, so as to avoid any pollution that may adversely affect the safety and quality of the next batch of products.

注：全文描述以中文为准。

Note: The full description is in Chinese.

- 范围Scope

本方案涵盖了与YY001原液产品接触、非产品接触的设备和器具的取样位置的描述、取样方法、可接受标准等内容。

This protocol includes the description of sampling locations, sampling methods and acceptable criteria of equipment and utensils of product contact and non-product of bulk drug substance YY001.

本方案不涵盖的内容：

The contents not covered in this protocol:

厂房、设施和HVAC管道的清洁效果不在本清洁验证方案范围内。

This cleaning validation protocol does not cover the cleaning effect of the factory buildings, facilities and HVAC pipes.

对于一次性使用的耗材不在此清洁验证方案内。非物料接触表面仅进行目视检查。

The disposable consumables are not included in this cleaning validation protocol. Only visual inspection is conducted for non-material contact surfaces.

清洁验证相关分析方法验证和清洁程序研究不在此清洁验证方案内。

Analysis method qualification and cleaning procedure research related to cleaning validation are not included in this cleaning validation protocol.

- 描述Description

本方案依据《注射用重组A型肉毒毒素清洁验证主计划》（文件编号：VMP-2024-02）和《注射用重组A型肉毒毒素清洁验证风险评估报告》（文件编号：QRM-2024-01-003）以及重庆誉颜相关文件制定。

This protocol is formulated according to Cleaning Validation Master Plan for Recombinant Botulinum Toxin A for Injection (Doc. No.: VMP-2024-02), Risk Assessment Report of Cleaning Validation for Recombinant Botulinum Toxin A for Injection (Doc. No.: QRM-2024-01-003) and relevant documents of Claruvis.

  - 纯化生产Purification Production

    - 工艺流程简述Brief Description of Process Flow

菌泥经过重悬、均质机破菌后、离心后取上清，通过5L不锈钢桶转移至50L储液袋中，在层析柜中预冷后上样GST层析柱，上样后加酶循环，收集酶切液，再次加酶静止酶切。

After the bacterial sludge is suspended for the second time, broken by a homogenizer, and centrifuged by centrifuge, the supernatant is taken, which is then transferred to a 50L solution storage bag through a 5L stainless steel barrel. After pre-cooling in a chromatography cabinet, the supernatant is loaded onto a GST chromatography column. Enzyme circulation is added after loading, and the enzymatic digestion solution is collected. Enzyme is added again for static enzymatic digestion.

酶切后加入酸沉淀溶液然后通过0.45μm过滤器过滤，获得CEX上样液，经过CEX层析、SEC层析收获SEC纯化液，最后经稀释、除菌过滤后获得原液。

After enzyme cut, acid precipitation solution was added, and then filtered by a 0.45μm filter. After filtration, the CEX loading solution was collected. This solution is then subjected to CEX chromatography and SEC chromatography to obtain the SEC purified solution. Finally, after dilution and sterile filtration, the bulk drug substance is obtained.

    - 工艺流程图Process Flow Chart

  - 设备、器具及相关清洁程序Equipment, Utensils and Related Cleaning Procedures

纯化车间属于专线用于YY001原液的生产。根据风险评估并分组确定的清洁验证目标设备、器具包括：所有使用的器具，高压均质系统、各层析工序的层析设备和层析柱、隔膜泵和上搅拌控制系统，评估过程详见《注射用重组A型肉毒毒素清洁验证风险评估报告》（文件编号：QRM-2024-01-003）。

The purification workshop belongs to a dedicated line for the production of bulk drug substance of YY001. According to the risk assessment and grouping, the target equipment and utensils for cleaning validation include: all utensils, high-pressure homogenization system, chromatography equipment and chromatography columns used in each chromatography process, diaphragm pumps and upper mixing control systems. See the Risk Assessment Report of Cleaning Validation for Recombinant Botulinum Toxin A for Injection (Doc. No.: QRM-2024-01-003) for details of the assessment process.

器具、设备清洁方式为CIP，COP或手动清洁方式，清洁流程及参数详见以下所列文件：

The cleaning methods for utensils and equipment are CIP, COP or manual cleaning, and the cleaning processes and parameters are detailed in the following documents:

| 序号 No. | 文件名称 Document Name | 文件编号 Document No. |
| --- | --- | --- |
|  | 器具清洗岗位（纯化）标准操作规程 Standard Operating Procedure for Utensils Cleaning (Purification) Position | SOP-3-YY001-009 |
|  | 均质机及配套罐系统使用、维护及清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of Homogenizer and Associated Tank System | SOP-3-EM-127 |
|  | APPS Process DN8s利穗自动层析系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of APPS Process DN8s LISURE Automatic Chromatography System | SOP-3-EM-174 |
|  | AKTA PILOT 600R层析系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of AKTA PILOT 600R Chromatography System | SOP-3-EM-133 |
|  | DDP03SWE/TC2/KT2型隔膜泵设备使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of DDP03SWE/TC2/KT2 Diaphragm Pump Equipment | SOP-3-EM-171 |
|  | 自动升降上搅拌系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of Automatic Lifting and Mixing System | SOP-3-EM-074 |

- 职责Responsibilities

本方案将由重庆誉颜制药有限公司和迈本医药科技有限公司相关人员共同完成，职责分配如下：

This protocol will be carried out by Claruvis and Mirable related personnel. Responsibilities are assigned as follows：

| 项目 Item | 迈本 MIRABLE | 重庆誉颜 Claruvis |
| --- | --- | --- |
| 参考文件资料提供 Relevant documents & information supply | N/A | S |
| 清洁验证方案编写 Cleaning validation protocol compiling | MR | S |
| 清洁验证方案审核/批准 Cleaning validation protocol review & approval | S | MR |
| 执行 Execution | S | MR |
| 数据整理 Data sorting | S | MR |
| 偏差管理 Deviation management | S | MR |
| 变更控制 Change control | S | MR |
| 清洁验证报告编写 Cleaning validation report compiling | MR | S |
| 清洁验证报告审核/批准 Cleaning validation report review & approval | S | MR |
| 备注Note：主负责Mainly responsible-MR；支持Support-S |  |  |

- 参考文件References

  - 法规和指南Regulations and Guidelines

- 中国，药品生产质量管理规范（2010年修订），2011年

- China, Good Manufacturing Practice (2010 Revision), 2011

- 中国，药品生产质量管理规范（2010年修订），附录：确认与验证,2015年

- China, Good Manufacturing Practice (2010 Revision), Appendix: Qualification and Validation, 2015

- 美国，21CFR Part 210：现行生产质量管理规范：生产，工艺，包装，储存，总则，现行版

- America, 21CFR Part 210- CURRENT GOOD MANUFACTURING PRACTICE IN MANUFACTURING, PROCESSING, PACKING, OR HOLDING OF DRUGS; GENERAL

- 美国，21CFR Part 211：现行生产质量管理规范，现行版

- America, 21CFR Part 211- CURRENT GOOD MANUFACTURING PRACTICE FORFINISHED PHARMACEUTICALS

- EudraLex 第四卷 欧盟药品生产质量管理规范指南 医药产品 人用及兽用

- EudraLex Volume 4 EU Guidelines to Good Manufacturing Practice Medicinal Products for Human and Veterinary Use

- EudraLex 第四卷 欧盟药品生产质量管理规范指南 医药产品 人用及兽用，附录15 确认与验证，2015年

- EudraLex Volume 4 EU Guidelines for Good Manufacturing Practice for Medicinal Products for Human and Veterinary Use, Annex 15: Qualification and Validation, 2015

- 中国药典（ChP），2020年

- Pharmacopoeia of the People's Republic of China 2020

- 美国药典（USP46 - NF41）

- U.S. Pharmacopeia National Formulary USP 46 NF41

- 欧洲药典（EP），EP 11.1

- EUROPEAN PHARMACOPOEIA 11.1

- 国际制药工程协会 指南 清洁验证生命周期 - 应用、方法和控制，2020年

- International Society for Pharmaceutical Engineering（ISPE） GUIDE:  Cleaning Validation Lifecycle – Applications, Methods, and Controls, 2020

- 注射剂协会 49号技术报告 生物技术清洁验证的考虑要点，2010

- Parenteral Drug Association Technical Reporter No.49 Point to Consider for Biotechnology Cleaning Validation, 2010

  - 文件Documents

| 序号 No. | 文件名称 Document Name | 文件编号 Document No. |
| --- | --- | --- |
|  | 确认与验证管理规程 Management Procedure for Qualification and Validation | SMP-3-QV-001 |
|  | 清洁验证管理规程 Management Procedure for Cleaning Validation | SMP-3-QV-011 |
|  | 人员培训管理规程 Management Procedure for Personnel Training | SMP-3-HR-002 |
|  | 偏差处理管理规程 Management Procedure for Deviation Handling | SMP-3-QA-005 |
|  | 变更控制管理规程 Management Procedure for Change Control | SMP-3-QA-007 |
|  | 记录管理规程 Management Procedure for Records | SMP-3-DM-003 |
|  | 文件控制程序 Procedure for Document Control | CP-2-DM-001 |
|  | 取样管理规程 Management Procedure for Sampling | SMP-3-QA-008 |
|  | 公用介质监测管理规程 Management Procedure for Utility Media Monitoring | SMP-3-QA-009 |
|  | 实验室异常管理规程 Management Procedure for Laboratory Abnormality | SMP-3-QC-019 |
|  | 超标（OOS）超趋势（OOT）结果处理管理规程 Management Procedure for Handling Out-of-Specification (OOS) and Out-of-Trend (OOT) Results | SMP-3-QC-020 |
|  | 表面微生物监测标准操作规程 Standard Operating Procedures for Surface Microbial Monitoring | SOP-3-QC-010 |
|  | 注射用重组A型肉毒毒素清洁验证主计划 Cleaning Validation Master Plan for Recombinant Botulinum Toxin A for Injection | VMP-2024-02 |
|  | 注射用重组A型肉毒毒素清洁验证风险评估报告 Risk Assessment Report of Cleaning Validation for Recombinant Botulinum Toxin A for Injection | QRM-2024-01-003 |
|  | 注射用重组A型肉毒毒素纯化工艺规程（草案） Purification Process Procedure for Recombinant Botulinum Toxin A for Injection (Draft) | TS-2-YY001-C |
|  | 器具清洗岗位（纯化）标准操作规程 Standard Operating Procedure for Utensils Cleaning (Purification) Position | SOP-3-YY001-009 |
|  | 均质机及配套罐系统使用、维护及清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of Homogenizer and Associated Tank System | SOP-3-EM-127 |
|  | APPS Process DN8s利穗自动层析系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of APPS Process DN8s LISURE Automatic chromatography System | SOP-3-EM-174 |
|  | AKTA PILOT 600R层析系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of AKTA PILOT 600R Chromatography System | SOP-3-EM-133 |
|  | DDP03SWE/TC2/KT2型隔膜泵设备使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of DDP03SWE/TC2/KT2 Diaphragm Pump Equipment | SOP-3-EM-171 |
|  | 自动升降上搅拌系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of Automatic Lifting and Mixing System | SOP-3-EM-074 |

- 验证指导Validation Guidelines

  - 测试项结构Structure of the Tests

本方案中涉及的验证测试项的结构如下：

The test items of validation involved in this protocol have the following structure：

- 测试项序号及名称 Test number and title

- 目的 Objective

- 程序 Procedures

- 接受标准 Acceptance criteria

- 结果 Results

- 结果判定Result Determination

- 备注 Comments

- 执行人员及复核人员签字 Executor and reviewer signature

  - 执行说明Execution Instructions

    - 执行原则 Execution Rules

- 执行测试前，按照“11 先决条件确认”确保该方案执行的先决条件已满足。

- Before executing the test, verify that the prerequisite for execution of the protocol is met in accordance with“11 Prerequisite Verification”.

- 按照方案中的测试程序执行每一个测试。在备注栏记录附加信息（附件、偏差、变更或其它说明）。若所收集的数据或证据证明验证结果符合方案中规定的要求可判定符合接受标准。

- Each test will be performed according to the established testing procedure. If any additional information （attachments, deviations, change or other instructions）needs to be documented, the comments section will be used. The acceptance criteria are considered to be met when the collected data or evidence demonstrated that the results of cleaning validation are consistent with the requirements in this protocol.

- 方案执行者应在测试记录表格中及确认人一栏中签名及日期。要求另一个有资质人员复核数据并确定测试被完整的执行，在复核人一栏中签名及日期。

- The person who executes this protocol will sign their name or initials and date in the test record table and “Verified by” column.  Another qualified person will review the data and verify that the test has been fully performed, and this person will sign their name or initials and date in the “Reviewed by”column.

    - 书写要求Handwriting Requirement

- 记录Recording

所有在此方案中填写数据的人员必须是在“签名&培训确认”表格中登记的人员；应签全名，除非文件另有规定；签名应始终一致。

Fill out all data in the protocol, the person must be registered in the "Signature &Training Verification" form; unless otherwise specified, the full name shall be used as the signature. The signatures shall be consistent.

使用蓝色签字笔记录，字迹清晰易读。

Blue sign pens shall be used for recording. Handwriting is legible.

填写栏目内容，应按照实际确认情况，对确认结果使用“√”填写，只能选择一种确认结果填写。不可以用“同上”等简写符号来替代记录或签名。

Fields shall be filled according to the actual verification result. The verification result shall be selected with “√” and only one result can be selected. You cannot use "ditto" or other shorthand symbols to replace the record or signature.

若有单个栏目不需要填入内容，则在空白处填写英文字母“不适用”的简写“N/A”，表示无此项内容。若有多个栏目不需要填入内容，应用斜线划掉，斜线上方填写“N/A” ，下方签名并注明日期。

If there is nothing to be filled into an individual field, N/A (abbreviation for not applicable) shall be written in this field. If there is nothing to be filled in more than one neighboring fields, such fields shall be crossed with a diagonal line, with “N/A” noted above the line, sign the name and date under the line.

举例如下

For example：

|  | N/A |  |  |
| --- | --- | --- | --- |
|  | 签名 日期 | Signature date |  |

使用规范的书面语及名称；文件前后名称要一致。

Standard written words and names shall be used; the names shall be consistent all through the document.

- 更改错误Correction of Mistakes

在错误处划单条删除线，填入正确的内容，签名并注明更改日期，确保原先信息仍清晰可识别。

When any mistakes are to be corrected, the correct record shall be entered, signed and dated for correction. The original information shall be left clearly readable.

如：2024年01月01日  修改人签名和修改日期

For example: January 1, 2024 signature and date

对于可追溯的修改原因（如：存在原始打印记录），必须在确认表格中的“备注”段落中阐明修改的理由并附上支持性的文件。

For traceable modification reasons (e.g. the original printed record is available), the modification reason must be explained in the column of comments in the verification table with supportive documents attached.

不可追溯的修改必须在确认表格中的“备注”段落中给予说明。

For non-traceable modification, explain the reason in the column of comments in the verification table.

复核人应对修改人的修改内容进行确认并在修改人签名和修改日期旁签署姓名和日期。

The reviewer should verify the modified content and sign the name and date beside the signature and date of the modifier.

- 日期及时间格式Date / Time Format

年用4位数表示，日和月用2位数表示

Four digits shall be used for the indication of the year, two digits for the month and another two for the date.

如：2024年01月01日

For example: 2024-01-01

使用24小时制格式来填写时间。

Use a 24-hour clock for recording time.

例如：上午 10:00 应为10:00

下午 2:00应为 14:00

E.g. 10:00 a.m. becomes 10:00 e.g. 2:00 p.m. becomes 14:00

- 数字记录规则Digital Recording Rule

数字记录后应附有单位，测试表格表头有单位备注除外。

Record the units for numerical data except for the test table header has unit notes

填写数据需考虑到小数点后的位数，该位数由测量仪表的精度决定，通常情况应考虑四舍五入，或取决于测试需求，在确认项中另行约定。

Fill in data to take into account the number of digits after the decimal point, which is determined by the accuracy of the measuring instrument. Normally, it should be considered rounded, or depending on the test requirements.

    - 附件 Attachment

执行过程中所有对测试报告数据进行支持的附件，如：打印数据等文件应附到报告中，并在“15 附件清单”中记录所有附件。

All attachments supporting test report data during execution, such as printing data, etc., should be attached to the report, and all attachments should be recorded in "15 Attachment List ".

对于每个附件，应包含如下信息：

For each attachment, the following information should be included:

- 附件编号，按流水号：附件 1，2，3……依次类推；

The attachment numbers. Example: Attachment 1, 2, 3, etc.；

- 附件名称；

Attachment title；

- 和附件对应的方案章节，如10.1 XXX确认；

The section number from the protocol, for which the attachment applies. Example: 10.1 XXX Verification ；

- 附件每页的页码及总页码，如1/4, 2/4 等；

The page number and number of pages in attachment. Example: 1/4, 2/4, etc.；

- 在附件的首页上签名和日期。

Signature and date in the first page of attachment.

    - 偏差处理Deviation Handling

如验证方案执行过程中出现任何不符合项，按照《偏差处理管理规程》（SMP-3-QA-005）进行偏差调查处理，在偏差报告中评估偏差对后续研究活动的影响，并在“13 偏差清单”中记录所有偏差。

Any non-conformity identified during the execution of validation protocol shall be investigation and handled following the “Management Procedure for Deviation Handling” (SMP-3-QA-005) and evaluate the impact of the deviation on subsequent validation activity in the deviation report, and all deviations should be recorded in "13 Deviation List ".

如检验过程中发生检验异常或超标，则按照《实验室异常管理规程》（SMP-3-QC-019）或《超标（OOS）超趋势（OOT）结果处理管理规程》（SMP-3-QC-020）进行调查。

If any abnormality or out-of-specification (OOS) occurs during the test process, an investigation shall be conducted in accordance with the “Management Procedure for Laboratory Abnormality” (SMP-3-QC-019) or the “Management Procedure for Handling Out-of-Specification (OOS) and Out-of-Trend (OOT) Results” (SMP-3-QC-020).

复印偏差调查报告附于验证报告后，异常调查报告和超标调查报告无需复印和附在本报告后。

Use signed and dated copies of the deviation report and attached it to validation report, while the abnormality investigation report and the out-of-specification investigation report do not need to be copied or attached to this report.

在验证报告中按照如下形式汇总所有的偏差及影响评估结果。

All the deviations and impact assessment results shall be summarized in the report of validation according to the following form.

| 偏差编号 Deviation No. | 对应的章节Related Section | 偏差描述 Deviation Description | 纠偏措施 Corrective Action | 状态 Status | 对验证影响 Assessment Conclusion of Impact of Validation |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ☐未关闭Unclosed ☐关闭Closed |  |
|  |  |  |  | ☐未关闭Unclosed ☐关闭Closed |  |

    - 变更控制Change Control

验证执行中如发生变更，需按照《变更控制管理规程》（SMP-3-QA-007）进行控制并提交报告，收集变更控制表，并在“14 变更清单”中记录所有变更。

If any changes are required during the validation, it should be controlled and reported in accordance with “Management Procedure for Change Control” (SMP-3-QA-007). All Change Control Forms shall be collected, and all changes should be recorded in "14 Change List ".

在验证报告中按照如下形式汇总所有的变更及影响评估结果。

All the changes and impact assessment results shall be summarized in the report of validation according to the following form.

| 变更编号 Change No. | 对应的章节 Related Section | 变更描述 Change Description | 状态 Status | 对验证影响 Assessment Conclusion of Impact of Validation |
| --- | --- | --- | --- | --- |
|  |  |  | ☐未关闭Unclosed ☐关闭Closed |  |
|  |  |  | ☐未关闭Unclosed ☐关闭Closed |  |

  - 验证报告Validation Report

执行本方案并将生成一份最终的验证报告。将编写对所获得结果的总结，并根据这些结果得出评价结论。

A final validation report will be generated once this protocol has been executed. A summary of the results obtained will be compiled and a conclusion is drawn based on these results.

验证报告将包括签批页、目的、描述、范围、描述（清洁验证中涉及的一些设备和器具清单及相关SOP）、经过验证的清洁工艺参数和范围、测试结果总结、偏差处理、变更控制、持续清洁确认和定期回顾的建议、最终验证的结论、测试记录、附件等组成。

The validation report will include the approval page, purpose, scope, description (List of equipment and utensils involved in cleaning validation and related cleaning SOP), verified cleaning process parameters and scope, test results summary, deviation handling, change control, suggestions for continuous cleaning qualification and regular review, final verification conclusion, test records, and attachments, etc.

  - 接受标准Acceptance Criteria

    - 清洁验证方案Cleaning Validation Protocol

在达到以下接受标准的情况下，该方案可以被认为已完成：

The protocol shall be considered as completion after having met the following acceptance criteria:

- 本次清洁验证需要完成连续三次成功的清洁验证，包括每次清洁验证的清洁操作已按照SOP要求准确执行，最差条件已进行挑战（在清洁参数要求下最可能造成清洁失败的参数和DHT）、CHT已按照SOP规定时限执行，相关的检测结果均符合验证标准要求。

- This cleaning validation requires the successful completion of three consecutive cleaning validations, including accurate execution of cleaning operations in accordance with SOP requirements for each validation, challenging of worst-case conditions (The parameter that is most likely to cause cleaning failure under the cleaning parameter requirements and DHT), execution of CHT within the prescribed time limit as per SOP, and all related test results meeting the validation criteria.

- 如果偏差和根本原因与清洁过程无关，则清洁验证的连续性可以不被认为是中断的。如果CHT验证失败，而DHT验证成功，则不认为是清洁工艺验证的失败，但需要重新设置、评估和验证CHT。

- If the deviation and root cause are extrinsic to the cleaning process, the continuity of cleaning validation may not be considered as disrupted. If the CHT validation fails while the DHT validation is successful, it is not considered a failure of the cleaning process validation, but the CHT needs to be reset, reassessed, and revalidated.

- 所有确认项目步骤都已经执行，并且结果已记录在测试表格中，各个步骤被认为通过或失败。

- All verification item steps have been executed and the results have been recorded in the test table, and each step is concluded as pass or fail.

- 已经附上所有支持确认结果的数据。

- All data supporting the verification results are attached.

- 所有数据页都由相关执行人和审核人署名及日期。

- All data Pages have been signed and dated by the relevant executor and reviewer.

- 确认所有记录都已完成。

- Verify that all records have been completed.

- 所有产生的偏差/变更已经得到解决和批准，偏差/变更报告已经附在文件中；或偏差/变更未关闭，但经评估不对验证活动产生重大影响。

- All deviations/ changes generated have been resolved and approved, deviation/ changes report had been attached in the document. Or deviations/ changes are not closed, but there is no significant impact on the validation activity by assessed.

    - 样品检验Sample Testing

本次清洁验证，各设备、器具将执行以下样品检验标准：

For this cleaning validation, all equipment and utensils will undergo the following sample testing criteria:

| 器具名称规格/设备名称编号 Name and specification of utensils, name and serial number of equipment |  | 取样阶段 Sampling Stage | 取样方法 Sampling method | 检验项目 Test items | 标准 Criteria |
| --- | --- | --- | --- | --- | --- |
| 制药用器具清洗干燥机 （用于器具的清洗） Cleaning and drying machine for pharmaceutical utensils (Used for cleaning utensils) | V-E111-22-002 | DHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |
|  |  |  |  | TOC | ≤0.5ppm (mg/L) |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤10cfu/100ml |
| 器具 Utensils | N/A | CHT | 擦拭取样 Swab sampling | 微生物限度Microbial limit | ≤25cfu/25cm2 |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) | V-E110-23-009 V-E110-23-010 | DHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |
|  |  |  |  | TOC | ≤0.5ppm (mg/L) |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤10cfu/100ml |
|  |  | CHT | 擦拭取样 Swab sampling | 微生物限度Microbial limit | ≤25cfu/25cm2 |
| 高压均质系统 High-pressure homogenization system | V-D121-22-003 | DHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |
|  |  |  |  | TOC | ≤5ppm (mg/L) |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |
|  |  |  | 擦拭取样 Swab sampling | TOC | ≤5ppm (mg/L) |
|  |  |  |  | 微生物限度Microbial limit | ≤25cfu/25cm2 |
|  |  | CHT | 擦拭取样 Swab sampling | 微生物限度Microbial limit | ≤25cfu/25cm2 |
| 全自动层析系统和层析柱 Fully automated chromatography system and chromatographic column | V-E116-22-016 V-E116-22-019 | DHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | TOC | ≤1ppm (mg/L) |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |
|  |  |  | 擦拭取样 Swab sampling | TOC | ≤1ppm (mg/L) |
|  |  |  |  | 微生物限度Microbial limit | ≤25cfu/25cm2 |
|  |  | CHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |
| AKTA层析系统和层析柱 （CEX层析） AKTA chromatography system and chromatographic columns (CEX chromatography) | V-E117-22-003 V-E117-22-007 V-E117-22-008 | DHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | TOC | ≤1ppm (mg/L) |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |
|  |  | CHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |
| AKTA层析系统和层析柱 （SEC层析） AKTA chromatography system and chromatographic columns (SEC chromatography) | V-E117-23-010 V-E117-22-004 V-E117-22-005 V-E117-22-006 | DHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | TOC | ≤2ppm (mg/L) |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/100ml |
|  |  | CHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/100ml |
| 隔膜泵（原液配制） Diaphragm pump (Bulk drug substance preparation) | V-E116-23-022 | DHT | 淋洗取样 Rinse sampling | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |
|  |  |  |  | 电导率Conductivity | ≤1.3μS/cm（25℃） |
|  |  |  |  | TOC | ≤1ppm (mg/L) |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/100ml |

- 验证策略Validation Strategy

  - 验证范围Validation Scope

本次清洁验证为用于生产YY001原液的纯化车间的首次清洁验证，采用结合《注射用重组A型肉毒毒素清洁验证主计划》的相关要求并基于风险的验证策略对本次清洁验证的程度和范围进行评估，根据评估结果开展清洁验证，风险评估过程及风险评估结果详见《注射用重组A型肉毒毒素清洁验证风险评估报告》。

This cleaning validation is the first cleaning validation of the purification workshop used to produce bulk drug substance of YY001. The degree and scope of this cleaning validation are evaluated by adopting the validation strategy based on the relevant requirements of the Cleaning Validation Master Plan for Recombinant Botulinum Toxin A for Injection, and the cleaning validation is carried out according to the evaluation results. See the Risk Assessment Report of Cleaning Validation for Recombinant Botulinum Toxin A for Injection for the risk assessment process and results for details.

根据《注射用重组A型肉毒毒素清洁验证主计划》、《注射用重组A型肉毒毒素清洁验证风险评估报告》确定纯化车间相关验证范围如下（DHT）：

According to Cleaning Validation Master Plan for Recombinant Botulinum Toxin A for Injection and Risk Assessment Report of Cleaning Validation for Recombinant Botulinum Toxin A for Injection, the relevant validation scope of purification workshop is determined as follows (DHT):

器具utensils:

| 器具名称规格 Name and specification of utensils |  | 数量 Quantity | 残留物 Residue | 检测方法和检测项目 Detection methods and detection Items |
| --- | --- | --- | --- | --- |
| 不锈钢桶 Stainless steel barrels | 5L | 4 | 破菌上清和NaOH。 Supernatant of bacteria-breaking solution and NaOH. | 目视检查、淋洗取样（性状、TOC、电导率、微生物限度和细菌内毒素）。 Visual inspection, rinse sampling (TOC, conductivity, microbial limit and bacterial endotoxin). |
| 玻璃棒 Glass bars | N/A | 3 | 各种缓冲液。 Various buffer solutions. |  |
| 烧杯 Beakers | 5L | 3 |  |  |
|  | 2L | 2 |  |  |
| 称量勺 Weighing spoons | N/A | 3 | 所有固体物料。 All of solid materials. |  |

设备Equipment:

| 设备名称编号 Name and serial number of equipment |  |  | 数量 Quantity | 残留物 Residue | 检测方法和检测项目 Detection methods and detection Items |
| --- | --- | --- | --- | --- | --- |
| 高压均质系统 High-pressure homogenization system) | 高压均质机 High-pressure homogenizer | V-D121-22-003 | 1 | 破菌液和NaOH。 Broken bacterial solution, and NaOH. | 目视检查、淋洗取样（性状、TOC、电导率（如适用）、微生物限度和细菌内毒素（如适用））。 Visual inspection, rinse sampling (TOC, conductivity, microbial limit and bacterial endotoxin). |
|  | 均质机排液管道 Homogenizer discharge pipeline |  |  |  |  |
|  | 收获罐来液管道 The incoming solution pipeline for the harvesting tank |  |  |  |  |
| 全自动层析系统 Fully automated chromatography system |  | V-E116-22-016 | 1 | 缓冲液、破菌液上清、循环酶切液和NaOH。 Buffer solution, supernatant of bacteria-breaking solution, circulating enzyme cut solution and NaOH. |  |
| AKTA层析系统 （CEX层析） AKTA chromatography system (CEX chromatography) |  | V-E117-22-003 | 1 | 缓冲液、CEX上样液、CEX纯化液和NaOH。 Buffer solution, CEX loading solution, CEX purified solution and NaOH. |  |
| 层析柱（CEX层析） Chromatographic columns (CEX chromatography) |  | V-E117-22-007 V-E117-22-008 | 2 |  |  |
| AKTA层析系统 （SEC层析） AKTA chromatography system (SEC chromatography) |  | V-E117-23-010 | 1 | 缓冲液、CEX纯化液、阳离子层析纯化液和NaOH。 Buffer solution, CEX purified solution, cation chromatography purification solution and NaOH. |  |
| 层析柱（SEC层析） Chromatographic columns (SEC chromatography) |  | V-E117-22-004 V-E117-22-005 V-E117-22-006 | 3 |  |  |
| 隔膜泵（原液配制） Diaphragm pump (Bulk drug substance preparation) |  | V-E116-23-022 | 1 | 原液稀释液和NaOH。 Dilution solution of bulk drug substance and NaOH. |  |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) |  | V-E110-23-009 | 1 | 各种缓冲液。 Various buffer solutions. |  |
|  |  | V-E110-23-010 | 1 |  |  |
| 100L收获罐 （高压均质系统） 100L Harvesting tank (High-pressure homogenization system) |  | V-D121-22-003 | 1 | 破菌液和NaOH。 Broken bacterial solution, and NaOH. | 目视检查、淋洗取样（性状、TOC、电导率、微生物限度和细菌内毒素）、擦拭取样（TOC和微生物限度）。 Visual inspection, rinse sampling (TOC, conductivity, microbial limit and bacterial endotoxin), swab sampling (TOC and microbial limit). |
| 层析柱 （GST亲和层析） Chromatographic column (GST affinity chromatography) |  | V-E116-22-019 | 1 | 缓冲液、破菌液上清、循环酶切液和NaOH。 Buffer solution, supernatant of bacteria-breaking solution, circulating enzyme cut solution and NaOH. |  |

  - 取样器具及取样量Sampling Instruments  and Number of Samples

为防止取样器具对样品造成污染，取样人员在取样操作前，应对取样器具的洁净状态的检查以及在使用/灭菌/清洁有效期进行必要的检查。

To prevent sampling instruments from contaminating the samples, sampling personnel should conduct necessary inspections of the cleanliness status of the sampling instruments as well as verifying that it is within the expiration date of use/sterilization/cleaning before sampling operations.

根据检验项目/取样方法以及《取样管理程序》（文件编号：SMP-3-QA-008），确定取样器具及取样量如下：

According to inspection items/sampling methods and Management Procedure for Sampling (Doc. No. SMP-3-QA-008), the sampling instruments and sampling size are determined as follows:

| 检测方法和检测项目 Detection Methods and Detection Items |  | 取样器具 Sampling Instruments | 取样量 Sample Size |
| --- | --- | --- | --- |
| 目视检查 Inspection Verification |  | —— | —— |
| 擦拭取样 Swab Sampling | TOC | 拭子、总有机碳样品瓶 Swabs, TOC sample bottles | 25cm2 |
|  | 微生物限度 Microbial Limit | 灭菌拭子、50ml无菌离心管 Sterilized swabs, 50ml Sterile centrifuge tubes | 25cm2 |
| 淋洗取样 Rinse Sampling | 性状 Properties | —— | —— |
|  | TOC | 总有机碳样品瓶 TOC sample bottles | 40ml*2 |
|  | 电导率 Conductivity | 蓝盖瓶 Blue-capped bottles | 100ml*2 |
|  | 细菌内毒素 Bacterial Endotoxin | 15ml无热原离心管 15ml non-pyrogenic centrifuge tubes | 10ml*2 |
|  | 微生物限度 Microbial Limit | 50ml无菌离心管 50ml Sterile centrifuge tubes | 40ml*2 |
|  |  | 灭菌后蓝盖瓶 Sterilized blue-capped bottles | 200ml*2 |

注：“*2”表示须取2份样品。

Note: "*2" indicates that 2 samples need to be taken.

  - 擦拭取样位置评估Swab Sampling location Evaluation

本方案中以较难清洁、持续接触产品、产品积聚、残留物积聚、窄点、较难接触、较难取样、污染风险、不规则性状、存水、气液界面、遮挡位置、最后清洁位置进行评估取样位置的评估，并进一步根据设备、器具的设计、清洁等评估最终取样位置并设置擦拭取样位置，并提供清洁设备和器具的代表性照片。

In this protocol, the sampling locations are evaluated based on hard to clean, constant product contact, product buildup, residue buildup, pinch point, hard to reach, hard to sample, contamination risk, irregular shape, can retain water, the air-liquid interface, occluded location and last locations to lose residue, and the final sampling locations are further evaluated according to the design and cleaning of equipment and utensils, and the swab sampling positions are set, and representative photos of cleaning equipment and utensils are provided.

擦拭取样位置评估详见附录1《擦拭取样位置评估》。

Refer to Annex 1 "Swab Sampling locations Evaluation" for details of the evaluation of swabbing sampling locations.

擦拭取样位置图例详见附录2《擦拭取样位置图例》。

Refer to Annex 2 "Illustration of Swabbing Sampling locations" for details of the Illustration of Swabbing Sampling locations.

  - 脏污保留时间和清洁保留时间Dirty Hold Time and Cleaning Hold Time

脏污保留时间（DHT）：是对清洗工艺的最差条件进行验证，是指生产/使用结束后至清洁开始的时间。

Dirty hold time (DHT): it is to verify the worst-case conditions of cleaning process, and it refers to the time from the end of production/ use to the beginning of cleaning.

由于残留物在设备和器具上停留的时间也会影响残留物清除的难度。新的残留物通常比表面干燥的残留物更容易清除，对清洁工艺构成了挑战，同时也可以为生产提供指导， 所以脏污保留时间（DHT）须在清洁验证中进行验证。

Because the time of residue staying on equipment and utensils will also affect the difficulty of residue removal. The new residue is usually easier to remove than the residue with dry surface, which poses a challenge to the cleaning process and can also provide guidance for production. Therefore, the dirty hold time (DHT) must be verified in the cleaning validation.

监测方法是：将使用后的设备、器具按照文件要求放置于相应位置并放置至规定时间后按照SOP进行清洁，清洁后进行相应取样及检测。

The monitoring method is as follows: put the used equipment and utensils in the corresponding position according to the requirements of the document, clean them according to SOP after they are put in the specified time, and conduct corresponding sampling and testing after cleaning.

清洁保留时间（CHT）：是指设备、器具在清洗结束后到灭菌前/使用前（如不进行灭菌）所存放的时间。

Cleaning hold time (CHT): refers to the storage time of equipment and utensils from the end of cleaning to before sterilization/use (if sterilization is not carried out).

为避免清洁后的存放导致微生物增加对产品的安全性风险、质量风险造成影响，同样，也为了避免因为微生物的增殖而对灭菌程序造成挑战，车间可根据不同设备、器具的使用情况来规定清洁设备、器具的存放时间，确保验证的存放时间对生产工艺的指导意义。CHT仅考虑由于CHT引起的微生物限度（必要时增加细菌内毒素）变化，同时进行目视检查。

In order to prevent the increase of microorganisms after cleaning from affecting the safety risk and quality risk of products, and also to avoid the challenge to sterilization procedures caused by the proliferation of microorganisms, workshop can specify the storage time of cleaning equipment and utensils according to the usage of different equipment and utensils, so as to ensure the guiding significance of the verified storage time to the production process. CHT only considers the changes of microbial limit (bacterial endotoxin may be added if necessary) caused by CHT, and also carries out visual inspection.

监测方法是：将清洁后的设备、器具按相应SOP保留至清洁保留时间（灭菌前和/或使用前）后，目视检查合格后对适用于在线/在位淋洗的设备、器具设备进行淋洗和取样，对于不适用于在线淋洗的，则需要手动加入一定量注射用水进行手动淋洗后取样。

The monitoring method is as follows: after the cleaned equipment and utensils are kept to the clean hold time (before sterilization and/or before use) according to the corresponding SOP, the equipment and utensils that are suitable for cleaning in place/ cleaning out place rinsing are rinsed and sampled after visual inspection; for those that are not suitable for online rinsing, a certain amount of water for injection needs to be manually added for manual rinsing and sampling.

如生产前进行前清洁且规定时限的，则CHT为前清洁时限。原液配制工序的隔膜泵在清洁后24小时内使用，不进行CHT验证。

If pre-cleaning is conducted before production and a specified time limit is set, then the CHT shall be the pre-cleaning hold time limit. The diaphragm pump in the bulk drug substance preparation process is to be used within 24 hours after cleaning, without the need for CHT validation.

纯化车间各设备、器具脏污保留时间和清洁保留时间见下表：

See the following table for dirty retention time and clean retention time of all equipment and appliances in the purification workshop:

| 车间 Workshop | 保留时间 Hold time |  |  |
| --- | --- | --- | --- |
| 纯化车间 Purification Workshop | 脏污保留时间 Dirty hold time | 生产结束/使用结束至清洁开始（期间有预清洁） Time from the end of production/usage to the start of cleaning (There is pre-cleaning during this period) | 器具、大搅拌桨和原液配制工序隔膜泵DHT为72小时，高压均质系统DHT为40小时（预清洁时限为＜30min）。 The DHT for the utensils, large paddles and diaphragm pump of bulk drug substance preparation process is 72 hours, the DHT for the high-pressure homogenization system is 40 hours(The pre-cleaning time limit is ＜30 minutes.). |
|  |  | 生产结束/使用结束至清洁开始（期间无预清洁） Time from the end of production/usage to the start of cleaning (No pre-cleaning during the period) | SEC层析工序AKTA层析系统和层析柱 DHT为24小时，CEX层析工序的 AKTA层析系统和层析柱的DHT为24小时，GST亲和层析工序的全自动层析系统含和层析柱的DHT为24小时。 The DHT for both the AKTA chromatography system and chromatographic columns used in the SEC chromatography process is 24 hours, the DHT for both the AKTA chromatography system and chromatography columns used in the CEX chromatography process is 24 hours, the DHT for both the fully automated chromatography system and chromatography column used in the GST affinity chromatography process is 24 hours. |
|  | 清洁保留时间 Clean hold time | 清洁结束至使用前 From the end of cleaning to before use | 器具、大搅拌桨和高压均质系统CHT均为5天，原液配制工序隔膜泵CHT为24小时。 The CHT for utensils, large paddles and high-pressure homogenization system is 5 days, the CHT for diaphragm pump of bulk drug substance preparation process is 24 hours. |
|  |  | 前清洁结束至使用前 From the end of pre-cleaning to before use | GST亲和层析工序的全自动层析系统和层析柱的CHT为24小时，CEX层析工序的 AKTA层析系统和层析柱的CHT为24小时，SEC层析工序AKTA层析系统和层析柱的CHT为12小时。 The CHT for both the fully automated chromatography system and chromatography column used in the GST affinity chromatography process is 24 hours, the CHT for both the AKTA chromatography system and chromatography columns used in the CEX chromatography process is 24 hours, the CHT for both the AKTA chromatography system and chromatography columns used in the SEC chromatography process is 12 hours. |

纯化车间器具由制药用器具清洗干燥机清洁（已经过设备确认，满载状态下无清洁死角），各器具接触的物料以及各器具本身清洁难度较低，所以在CHT时选择相对具有代表性的烧杯杯口和不锈钢桶桶口。

The utensils in the purification workshop are cleaned by a cleaning and drying machine for pharmaceutical utensils (which has been verified through equipment verification to have no cleaning dead zones under full load conditions). The materials in contact with each piece of utensils and the utensils themselves are relatively easy to clean. Therefore, during the CHT process, we selected relatively representative items for microbial limit sampling through CHT swabbing: the neck of the beakers and the neck of the stainless steel barrels.

  - 样品编号原则Principles for Sample Numbering

    - 编号形式 Numbering Format

- 对清洁验证中取样的每个样品进行编号和标记，以避免样品混淆。编号原则为：验证次数（1/2/3）+设备/器具代码+D（表示DHT）/C（表示CHT）+序号（1/2/3……）。

- During the cleaning validation, each sample taken should be numbered and labeled to prevent confusion. The numbering principle is as follows: Validation Round (1/2/3) + Equipment/ Utensils Code + D (indicating DHT) / C (indicating CHT) + Serial Number (1/2/3...).

    - 设备/器具代码Equipment/ Utensils Code

| 设备名称编号/器具名称规格 Name and serial number of equipment, name and specification of utensils |  |  | 设备/器具代码 Equipment/ Utensils Code |
| --- | --- | --- | --- |
| 制药用器具清洗干燥机 Cleaning and drying machine for pharmaceutical utensils |  | V-E111-22-002 | C1 |
| 烧杯 Beakers |  | 5L | C2 |
| 不锈钢桶 Stainless steel barrels |  | 5L | C3 |
| 上搅拌控制系统（大搅拌桨） Upper mixing control systems (Large mixing paddles) |  | V-E110-23-009 V-E110-23-010 | C4 |
| 高压均质系统 High-pressure homogenization system | 高压均质机 High-pressure homogenizer | V-D121-22-003 | C5 |
|  | 收获罐来液管道 The incoming solution pipeline for the harvesting tank |  | C6 |
|  | 100L收获罐 100L Harvesting tank |  | C7 |
|  | 均质机排液管道 Homogenizer discharge pipeline |  | C8 |
| 全自动层析系统和层析柱 Fully automated chromatography system and chromatographic column |  | V-E116-22-016 V-E116-22-019 | C9 |
| AKTA层析系统和层析柱 （CEX层析） AKTA chromatography system and chromatographic columns (CEX chromatography) |  | V-E117-22-003 V-E117-22-007 V-E117-22-008 | C10 |
| AKTA层析系统和层析柱 （SEC层析） AKTA chromatography system and chromatographic columns (SEC chromatography) |  | V-E117-23-010 V-E117-22-004 V-E117-22-005 V-E117-22-006 | C11 |
| 隔膜泵（原液配制） Diaphragm pump (Bulk drug substance preparation) |  | V-E116-23-022 | C12 |

    - 样品编号Sample Numbering

各器具、设备清洁验证样品编号详见附录3《样品编号表》。

For detailed sample numbering of cleaning validation for various utensils and equipment, please refer to Annex 3 Sample Number Table.

  - 分析方法和取样回收率Analysis Method and Sampling Recovery Factor

本次清洁验证执行中，针对残留物、电导率、清洁剂、细菌内毒素和微生物限度设定验证标准并采用适当的分析方法进行检验。评估过程详见《注射用重组A型肉毒毒素清洁验证风险评估报告》（文件编号：QRM-2024-01-003）.

During the execution of this cleaning validation, validation criteria has been set for residues, conductivity, cleaning agents, bacterial endotoxin, and microbial limits, and appropriate analytical methods have been used for testing. Details of the evaluation process are outlined in the Risk Assessment Report of Cleaning Validation for Recombinant Botulinum Toxin A for Injection (Doc. No.: QRM-2024-01-003).

残留物主要通过TOC检测法检测，清洁剂的残留通过电导率检测法检测，细菌内毒素采用细菌内毒素检测法检测，微生物限度通过微生物限度检测法检测。

The residues are mainly detected through the TOC detection method, the residual cleaning agents are detected through the conductivity detection method, the bacterial endotoxin is detected using the bacterial endotoxin detection method, and the microbial limit is detected through the microbial limit detection method.

另外，在取样回收率研究符合预期要求的情况下，除TOC值标准为≤0.5ppm的样品和纯化车间SEC层析工序的AKTA层析系统和层析柱采用TOC值≤2ppm且不使用回收率校准（层析柱填料为重复使用）外的TOC擦拭取样回收率和TOC淋洗取样回收率均采用50%，则TOC值≤10ppm的标准经50%取样回收率校正后换算为≤5ppm，TOC值≤2ppm的标准经50%取样回收率校正后换算为≤1ppm。

In addition, under the condition that the sampling recovery factor meets the expected requirements, with the exception of samples with a TOC value criteria of ≤0.5ppm and the AKTA chromatography system and chromatography columns used in the SEC chromatography process in the purification workshop, which adopt a TOC value of ≤2ppm and do not use recovery factor calibration (the chromatography column resin is reused), the TOC swab sampling recovery factor and TOC rinse sampling recovery factor are both set at 50%. Therefore, the criteria of ≤10ppm for TOC values, after being corrected with a 50% sampling recovery factor, is converted to ≤5ppm, and the criteria of ≤2ppm for TOC values, after being corrected with a 50% sampling recovery factor, is converted to ≤1ppm.

- 注意事项Precautions

- 岗位操作人员应严格按照清洁设备操作SOP以及设备、器具的清洁SOP进行清洁操作。

- Operators of the position should strictly follow the SOP for cleaning equipment and SOP of cleaning equipment and utensils to perform cleaning operations.

- 采用手动清洁或设备清洁但仍需人工辅助判定、计时等进行清洁操作的设备、器具时（如适用），如果SOP中规定的清洁工艺参数不是固定值，而是范围（例如浸泡时间、浸泡温度、淋洗时间、淋洗温度、清洁剂浓度、清洁剂温度、人工判定的清洁终点等），则岗位操作人员在清洁操作过程中应选择最差条件，即在清洁参数要求下最可能造成清洁失败的参数。

- When manually cleaning or using equipment to clean but still requiring manual assistance for judgment, timing, and other cleaning operations for equipment and utensils (if applicable), if the cleaning process parameters specified in the SOP are not fixed values but ranges (such as soaking time, soaking temperature, rinsing time, rinsing temperature, cleaning agent concentration, cleaning agent temperature, manually judged cleaning endpoint, etc.), the operators should select the worst-case conditions during the cleaning process, which refer to the parameters that are most likely to lead to cleaning failure under the cleaning parameter requirements.

- 清洁验证执行时，岗位操作人员与取样人员应密切沟通，确保在最后一次设备、器具淋洗时，及时取样。

- During the cleaning validation, the position operator should timely communicate with the sampling personnel to ensure that the samples are taken in time during the last equipment and utensils rinsing.

- 清洁验证执行时，岗位操作人员与取样人员应密切沟通，取样人员须提前准备相关的取样器具，并根据相关的管理要求及流程传入洁净区内，并完成必要的取样器具的检查和/或准备。

- During the implementation of cleaning validation, the position operator should timely communicate with the sampling personnel, who should prepare relevant sampling instruments in advance and transfer them into the clean area in accordance with relevant management requirements and procedures, and complete the necessary inspection and/or preparation of the sampling instruments.

- 如使用额外的淋洗水进行淋洗取样时，清洁操作人员应使淋洗水充分淋洗所有物料接触表面。

- If additional rinsing water is used for rinsing sampling, the cleaning operator shall make the rinsing water fully rinse all the materials contacting surfaces.

- 如待取样设备、器具既有淋洗取样又有擦拭取样的取样位置，应先进行淋洗取样。由于本次清洁验证淋洗水均为注射用水，则样品取样顺序为：TOC、其他理化项目（性状、电导率）、细菌内毒素，最后是微生物限度。

- If the equipment and utensils to be sampled have sampling locations for both rinse sampling and swab sampling, rinse sampling should be carried out first. As the rinsing water used for this cleaning validation is water for injection, the sample collection order is as follows: TOC, other physicochemical items (properties, conductivity), bacterial endotoxin, and finally microbial limit.

- 取样操作人员应保证取样器具应准备必要的备用，避免因容器倾倒、污染等异常情况导致取样器具不足影响取样。

- The sampling operator shall ensure that the sampling instruments should be prepared for necessary backup, so as to avoid the sampling being affected by the shortage of sampling instruments due to abnormal conditions such as container dumping and pollution.

- 取样人员在取样操作过程中，必须佩戴无菌手套和口罩。同时应避免身体接触任何待取样位置，以避免人员对待取样表面的污染，同时也应当避免因对取样器具造成污染而导致的样品污染。

- Sampling personnel must wear sterile gloves and masks during sampling operation. At the same time, physical contact with any position to be sampled should be avoided, so as to avoid the pollution of the surface to be sampled by personnel, and at the same time, the sample pollution caused by the pollution of sampling instruments should be avoided.

- 如待取样表面既有微生物限度取样，也有残留物擦拭取样，应先进行微生物限度擦拭取样，然后进行残留物擦拭取样。

- If there are both microbial limit sampling and residue swab sampling on the surface to be sampled, microbial limit swab sampling should be carried out first, and then residue swab sampling should be carried out.

- 如清洁验证中使用接触碟进行表面微生物限度取样，则采样结束后，立即用蘸有消毒剂的洁净抹布擦拭被取样表面，以去除残留琼脂。

- If contact plates are used for surface microbial limit sampling during cleaning validation, the sampled surface should be wiped immediately after sampling with a clean cloth dipped in disinfectant to remove residual agar.

- 取样操作人员如在取样前怀疑取样器具已受到污染，应立即更换。

- If the sampling operator suspects that the sampling instruments have been contaminated before sampling, he should replace it immediately.

- 取样操作人员在擦拭取样时，应避免微生物限度擦拭取样和残留物擦拭取样在擦拭表面有重叠。

- When swabbing samples, sampling operators should avoid the overlap between microbial limit swabbing samples and residue swabbing samples on the swabbing surface.

- 取样操作人员在进行清洁保留时间取样时，应避免已在DHT期间的取样位置取样。

- Sampling operators should avoid sampling at the sampling location during DHT when sampling with clean hold time.

- 取样操作人员在进行擦拭取样时，如擦拭面积不满足方案要求的取样面积时，须在记录实际的取样面积，并根据检验结果折算至规定的取样面积下的限度值。

- When sampling, if the swabbing area does not meet the sampling area required by the protocol, the sampling operator shall record the actual sampling area and convert it to the limit value under the specified sampling area according to the inspection results.

- 本次清洁验证的擦拭样品检验过程中无需额外扣除拭子空白样品和溶剂空白样品的TOC值，可提供额外的安全因子。

- During the inspection process of swab samples for this cleaning validation, there is no need to deduct the TOC values of the swab blank samples and solvent blank samples, which provides an additional safety factor.

- 取样人员在取样后应妥善保存样品避免因人员、环境等因素（例如样品容器密封不严、转移过程中发生倾倒等）对样品造成潜在污染。

- Sampling personnel should properly preserve the samples after sampling to avoid potential pollution to the samples due to personnel, environment and other factors (such as poor sealing of sample containers, dumping during transfer, etc.).

- QC分析人员应按照相关SOP和经验证/确认的分析方法对样品进行保存、处理和检验，确保检验结果准确。

- QC analysts should store, process, and test samples in accordance with relevant SOPs and validated/qualified analytical methods to ensure accurate inspection results.

- 测试项列表Tests List

本方案包括以下测试项目：

The protocol includes the following test items:

| 序号 No. | 测试项名称 Test Title | 目的 Purpose | 章节号 Section No. |
| --- | --- | --- | --- |
| 先决条件确认 Prerequisite Verification |  |  |  |
|  | 签名&培训确认 Signature &Training Verification | 确认在方案实施前针对执行人员进行了本方案的培训，培训记录存在，培训效果合格。 To verify all personnel involved in the execution of this protocol have been trained before performing the protocol, the training records are available and the training effect are qualified. 残留物擦拭取样操作人员具有资质。 Operators conducting residue swab sampling should be qualified. | 11.1 |
|  | 文件签批确认 Document Approval Verification | 确认清洁验证执行相关文件均已批准，本方案已批准。 To verify that all documents related to the execution of cleaning validation have been approved, and this protocol has been approved. | 11.2 |
|  | 设备确认文件确认 Equipment Qualification Document Verification | 确认清洁验证相关的生产设备、清洁设备已经过确认，并在有效期内。 To verify that the production equipment and cleaning equipment related to cleaning validation have been verified and are within the expiration date. | 11.3 |
|  | 公用系统确认文件确认 Utilities Qualification Document Verification | 确认清洁验证相关的公用系统已经过确认，并在有效期内。 To verify that the utilities related to cleaning validation have been verified and are within the expiration date. | 11.4 |
|  | 分析方法验证文件确认 Analysis Methods Validation Document Verification | 确认清洁验证相关的分析方法已经过确认，并在有效期内。 To verify that the analysis methods related to cleaning validation have been validated and are within the expiration date. | 11.5 |
|  | 测试仪器仪表校准确认 Test Instruments Calibration Verification | 确认所有在执行过程中用到的检验仪器、仪表、衡器都经过校准，并在有效期内。 To verify all inspection instruments, gauges and weighing apparatus used in execution process are calibrated and within the expiration date. | 11.6 |
|  | 测试仪器仪表确认文件确认 Test Instruments Qualification Document Verification | 所有在本方案执行过程中用到的检验仪器（如适用）、仪表（如适用）、衡器（如适用）都应是经过确认的，并确保在有效期内。 All the inspection instruments (if applicable), gauges (if applicable) and weighing apparatus (if applicable) used during the implementation of this protocol should be qualified, and ensure they are within the expiration date. | 11.7 |
| 清洁验证测试项目 Test Items of Cleaning Validation |  |  |  |
|  | 清洁操作及目视检查确认 Cleaning Operation and Visual Inspection Verification | 确认脏污保留至规定时间后，设备、器具已按照SOP进行清洁操作，清洁操作参数已完整记录和/或打印，且目视符合可接受标准。 To verify that after the dirt is kept for the specified time, that the equipment and utensils have been cleaned according to SOP, and that the cleaning operation parameters have been completely recorded and/or printed and results of visual inspection meet the acceptance criteria. | 12.1 |
|  | DHT淋洗取样确认 DHT Rinse Sampling Verification | 确认脏污保留至规定时间并完成清洁操作后，淋洗取样结果符合可接受标准要求。 To verify that the rinse sampling results meet the requirements of acceptable criteria after retaining dirt for the specified time and completing the cleaning operation. | 12.2 |
|  | DHT擦拭取样TOC确认 DHT Swab Sampling（TOC） Verification | 确认脏污保留至规定时间并完成清洁操作后，擦拭取样TOC结果符合可接受标准要求。 To verify that the swab sampling TOC results meet the requirements of acceptable criteria after retaining dirt for the specified time and completing the cleaning operation. | 12.3 |
|  | DHT擦拭取样微生物限度确认 DHT Swab Sampling Microbial Limit Verification | 确认脏污保留至规定时间并完成清洁操作后，擦拭取样微生物限度结果符合可接受标准要求。 To verify that the swabbing sampling microbial limit results meet the requirements of acceptable criteria after retaining dirt for the specified time and completing the cleaning operation. | 12.4 |
|  | CHT目视检查确认 CHT Visual Inspection Verification | 确认清洁后或前清洁后保留时间保留至规定时间后，设备、器具物料接触表面目视符合可接受标准。 To verify that the retention time after cleaning or after pre-cleaning is maintained until the specified time, the materials contact surfaces of equipment and utensils meets the acceptable criteria visually. | 12.5 |
|  | CHT淋洗取样确认 CHT Rinse Sampling Verification | 确认前清洁后保留时间保留至规定时间后，淋洗取样结果符合可接受标准要求。 To verify that the hold time after pre-cleaning is maintained until the specified time, the rinsing sampling results meet the acceptable criteria. | 12.6 |
|  | CHT擦拭取样确认 CHT Swab Sampling Verification | 确认清洁后或前清洁后保留时间保留至规定时间后，擦拭取样结果符合可接受标准要求。 To verify that the hold time after cleaning or after pre-cleaning is maintained until the specified time, the swab sampling results meet the acceptable criteria. | 12.7 |

- 先决条件确认Prerequisite Verification

在进行清洁验证之前首先进行以下先决条件确认，确认已经具备执行验证的条件。

The following prerequisites should be verified to verify to the execution conditions before the cleaning validation, and verify that the conditions for validation have been met.

  - 签名&培训确认Signature &Training Verification

为确定执行本方案的人员身份，应在执行前记录他们的名字、公司、部门/职务等信息，并由本人在签名处用蓝色签字笔书写本人姓名以及日期。

To enable the identification of personnel associating with performing this protocol is required to record their name, company, department/title etc., and sign the name and date in person with blue pen.

在方案实施前确保所有执行人员已经过培训，培训记录存在，培训效果合格。

Ensure that all personnel involved in the execution of this protocol have been trained before performing the protocol, the training records are available and the training effect is qualified.

在备注处备注执行残留物擦拭取样人员。

In the comments column, note the person who performed the residue swab sampling.

| 页数Page of___ |  |  |  |  |
| --- | --- | --- | --- | --- |
| 培训主题 Training Topic |  |  |  |  |
| 培训人 Trainer |  |  |  |  |
| 培训日期 Training Date |  |  |  |  |
| 培训地点 Training Location |  |  |  |  |
| 姓名 Name | 公司 Company | 部门/职位 Department/ Title | 签名 Signature | 培训效果 Training Result |
|  |  |  |  | □合格Pass □不合格Fail |
|  |  |  |  | □合格Pass □不合格Fail |
|  |  |  |  | □合格Pass □不合格Fail |
|  |  |  |  | □合格Pass □不合格Fail |
|  |  |  |  | □合格Pass □不合格Fail |
|  |  |  |  | □合格Pass □不合格Fail |
|  |  |  |  | □合格Pass □不合格Fail |
|  |  |  |  | □合格Pass □不合格Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                            □是Yes     □否No

确认人/日期Verified by/Date                            复核人/日期Reviewed by /Date

  - 文件签批确认Document Approval Verification

在清洁验证执行前，确认下列文件已签批。下面表格为文件签批确认表：

Verify that the following program has been signed and approved prior to cleaning validation execution. The following table shows the document sign-off verification form.

| 页数Page of___ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 序号 No. | 文件名称 Document Name | 文件编号 Document No. | 版本 Version | 批准日期 Approved Date | 通过/失败 Pass / Fail |
|  | 注射用重组A型肉毒毒素清洁验证主计划 Cleaning Validation Master Plan for Recombinant Botulinum Toxin A for Injection | VMP-2024-02 |  |  | □ 通过Pass □ 失败Fail |
|  | 注射用重组A型肉毒毒素清洁验证风险评估报告 Risk Assessment Report of Cleaning Validation for Recombinant Botulinum Toxin A for Injection | QRM-2024-01-003 |  |  | □ 通过Pass □ 失败Fail |
|  | 取样回收率研究报告 Study Report of Sampling Recovery Factor | VR-MV-0030-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 注射用重组A型肉毒毒素原液纯化阶段清洁验证方案 Cleaning Validation Protocol for the Purification Stage of Recombinant Botulinum Toxin A for Injection Bulk Drug Substance | VP-CV-0002-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 器具清洗岗位（纯化）标准操作规程 Standard Operating Procedure for Utensils Cleaning (Purification) Position | SOP-3-YY001-009 |  |  | □ 通过Pass □ 失败Fail |
|  | 均质机及配套罐系统使用、维护及清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of Homogenizer and Associated Tank System | SOP-3-EM-127 |  |  | □ 通过Pass □ 失败Fail |
|  | APPS Process DN8s利穗自动层析系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of APPS Process DN8s LISURE Automatic chromatography System | SOP-3-EM-174 |  |  | □ 通过Pass □ 失败Fail |
|  | AKTA PILOT 600R层析系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of AKTA PILOT 600R Chromatography System | SOP-3-EM-133 |  |  | □ 通过Pass □ 失败Fail |
|  | DDP03SWE/TC2/KT2型隔膜泵设备使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of DDP03SWE/TC2/KT2 Diaphragm Pump Equipment | SOP-3-EM-171 |  |  | □ 通过Pass □ 失败Fail |
|  | 自动升降上搅拌系统使用、维护、清洁标准操作规程 Standard Operating Procedure for Use, Maintenance, and Cleaning of Automatic Lifting and Mixing System | SOP-3-EM-074 |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                             □是Yes     □否No

确认人/日期Verified by/Date                                            复核人/日期Reviewed by /Date

  - 设备确认文件确认Equipment Qualification Document Verification

确认清洁验证相关的生产设备、清洁设备已经过确认，并在有效期内。下面表格为文件签批确认表：

To verify that the production equipment and cleaning equipment related to cleaning validation have been verified and are within the expiration date. The following table shows the document sign-off verification form.

| 页数Page of___ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 序号 No. | 文件名称 Document Name | 设备编号 Equipment No. | 报告编号 Report No. | 版本 Version | 有效期至 Expiration Date | 通过/失败 Pass / Fail |
|  | YQG-D-V-0.9 -H 型制药用器具清洗干燥机（纯化车间）性能确认（PQ）报告 Performance Qualification (PQ) Report for YQG-D-V-0.9 -H Cleaning and Drying Machine for Pharmaceutical Utensils (Purification Workshop) | V-E111-22-002 | VR（PQ）-EM-0034-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 100L发酵系统（发酵车间）性能确认（PQ）报告 （含CIP站确认内容） Performance Qualification (PQ) Report for 100L Fermentation System (Fermentation Workshop) (including CIP station validation content) | V-D121-22-001 | VR（PQ）-EM-0011-01 |  |  | □ 通过Pass □ 失败Fail |
|  | AH12-150 型高压均质机系统（纯化车间）性能确认（PQ）报告 Performance Qualification (PQ) Report for AH12-150 High-Pressure Homogenizer System (Purification Workshop) | V-D121-22-003 | VR（PQ）-EM-0013-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 上搅拌控制系统1安装/运行确认（IOQ）报告 Installation and Operation Qualification (IOQ) Report for Upper Mixing Control System 1 | V-E110-23-009 | VR（IOQ）-EM-0107-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 上搅拌控制系统2安装/运行确认（IOQ）报告 Installation and Operation Qualification (IOQ) Report for Upper Mixing Control System 2 | V-E110-23-010 | VR（IOQ）-EM-0108-01 |  |  | □ 通过Pass □ 失败Fail |
|  | APPS Process DN8s 型全自动层析系统（纯化车间）性能确认（PQ）报告 Performance Qualification (PQ) Report for APPS Process DN8s Fully Automatic Chromatography System (Purification Workshop) | V-E116-22-016 | VR（PQ）-EM-0094-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 手动螺杆压缩层析柱（纯化车间）安装/运行确认（IOQ）报告 Installation and Operation Qualification (IOQ) Report for Manual Screw Compression Chromatography Column (Purification Workshop) | V-E116-22-019 | VR03(IOQ)-EM-0098-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 智能泵过滤系统2安装/运行确认（IOQ）报告 Installation and Operation Qualification (IOQ) Report for Smart Pump Filtration System 2 | V-E116-23-022 | VR（IOQ）-EM-0101-01 |  |  | □ 通过Pass □ 失败Fail |
|  | pilot600 型AKTA层析系统1（纯化车间）性能确认（PQ）报告 Performance Qualification (PQ) Report for pilot600 AKTA Chromatography System 1 (Purification Workshop) | V-E117-22-003 | VR（PQ）-EM-0030-01 |  |  | □ 通过Pass □ 失败Fail |
|  | pilot600 型AKTA层析系统2（纯化车间）性能确认（PQ）报告 Performance Qualification (PQ) Report for pilot600 AKTA Chromatography System 2 (Purification Workshop) | V-E117-23-010 | VR（PQ）-EM-0081-01 |  |  | □ 通过Pass □ 失败Fail |
|  | BPG100/750型SEC层析柱1（纯化车间）安装/运行确认（IOQ）报告 Installation/Operational Qualification (IOQ) Report for BPG100/750 SEC Chromatography Column 1 (Purification Workshop) | V-E117-22-004 | VR（IOQ）-EM-0095-01 |  |  | □ 通过Pass □ 失败Fail |
|  | BPG100/750型SEC层析柱2（纯化车间）安装/运行确认（IOQ）报告 Installation/Operational Qualification (IOQ) Report for BPG100/750 SEC Chromatography Column 2 (Purification Workshop) | V-E117-22-005 | VR（IOQ）-EM-0096-01 |  |  | □ 通过Pass □ 失败Fail |
|  | BPG100/750型SEC层析柱3（纯化车间）安装/运行确认（IOQ）报告 Installation/Operational Qualification (IOQ) Report for BPG100/750 SEC Chromatography Column 3 (Purification Workshop) | V-E117-22-006 | VR（IOQ）-EM-0097-01 |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                             □是Yes     □否No

确认人/日期Verified by/Date                                            复核人/日期Reviewed by /Date

  - 公用系统确认文件确认Utilities Qualification Document Verification

确认清洁验证相关的公用系统已经过确认，并在有效期内。下面表格为文件签批确认表：

To verify that the utilities related to cleaning validation have been verified and are within the expiration date. The following table shows the document sign-off verification form.

| 页数Page of___ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 序号 No. | 文件名称 Document Name | 设备编号 Equipment No. | 报告编号 Report No. | 版本 Version | 有效期至 Expiration Date | 通过/失败 Pass / Fail |
|  | 空气净化系统（纯化车间）变更再确认（RQ）报告 Re-qualification (RQ) Report for Change of HVAC (Purification Workshop) |  | VR（RQ01）-FM-0010-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 纯化水系统 2023年度质量回顾分析报告 Quality Review and Analysis Report for Purified Water System in 2023 |  | QR-01-2023-06 |  |  | □ 通过Pass □ 失败Fail |
|  | 注射用水系统2023年度质量回顾分析报告 Quality Review and Analysis Report for Water for Injection System in 2023 |  | QR-01-2023-12 |  |  | □ 通过Pass □ 失败Fail |
|  | 工艺气体系统2023年度质量回顾分析报告 Quality Review and Analysis Report for Process Air System in 2023 |  | QR-01-2024-05 |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                             □是Yes     □否No

确认人/日期Verified by/Date                                            复核人/日期Reviewed by /Date

  - 分析方法验证文件确认Analysis Methods Validation Document Verification

确认清洁验证相关的分析方法已经过确认，并在有效期内。下面表格为文件签批确认表：

To verify that the analysis methods related to cleaning validation have been validated and are within the expiration date. The following table shows the document sign-off verification form.

| 页数Page of___ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 序号 No. | 文件名称 Document Name | 文件编号 Document No. | 版本 Version | 有效期至 Expiration Date | 通过/失败 Pass / Fail |
|  | 注射用重组A型肉毒毒素原液配制液微生物限度检查方法适用性确认报告 Applicability Qualification Report for the Microbial Limit Test Method of Preparation Solution of Recombinant Botulinum Toxin A for Injection Bulk Drug Substance | VR-MV-0012-01 |  |  | □ 通过Pass □ 失败Fail |
|  | 注射用水细菌内毒素检查方法学验证报告 Validation Report for the Methodology for the Test of Bacterial Endotoxin in Water for Injection | VR-MV-0003-01 |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                             □是Yes     □否No

确认人/日期Verified by/Date                                            复核人/日期Reviewed by /Date

  - 测试仪器仪表校准确认Test Instruments Calibration Verification

所有在本方案执行过程中用到的检验仪器、仪表、衡器都应是经过国家计量部门检定合格的或自行校准的，并确保在有效期内。将仪器仪表信息填入下表中。

All the inspection instruments, gauges and weighing apparatus used during the implementation of this protocol should be identified by national metrology department or calibrated by self, and ensure they are within the expiration date. Fill the information of instruments and apparatus in the table below.

| 页数Page of___ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 序号 No. | 仪器仪表名称 Instruments Name | 型号 Model | 编号 Serial No. | 校准证书编号 Certification No. | 有效期至 Expiration Date | 通过/失败 Pass / Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                             □是Yes     □否No

确认人/日期Verified by/Date                                            复核人/日期Reviewed by /Date

  - 测试仪器仪表确认文件确认Test Instruments Qualification Document Verification

所有在本方案执行过程中用到的检验仪器仪表（如适用）、仪表（如适用）、衡器（如适用）都应是经过确认的，并确保在有效期内。下面表格为文件签批确认表：

All the inspection instruments (if applicable), gauges (if applicable) and weighing apparatus (if applicable) used during the implementation of this protocol should be qualified, and ensure they are within the expiration date. The following table shows the document sign-off verification form.

页数Page    of___

| 序号 No. | 文件名称 Document Name | 设备编号 Equipment No. | 报告编号 Report No. | 版本 Version | 有效期至 Expiration Date | 通过/失败 Pass / Fail |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                             □是Yes     □否No

确认人/日期Verified by/Date                                            复核人/日期Reviewed by /Date

- 清洁验证测试项目Test Items of Cleaning Validation

  - 清洁操作及目视检查确认Cleaning Operation and Visual Inspection Verification

目的/ Objective

- 确认脏污保留至规定时间后，设备、器具已按照SOP进行清洁操作，清洁操作参数已完整记录和/或打印，目视符合可接受标准。 To verify that after the dirt is kept for the specified time, that the equipment and utensils have been cleaned according to SOP, and that the cleaning operation parameters have been completely recorded and/or printed. Results of visual inspection meet the acceptance criteria.

程序/Procedures

- 生产结束/使用结束后，按8.4所列设备、器具脏污保留时间保留至规定时间后，开始清洁操作。时间的记录应包括年、月、日、时和分。其中高压均质系统还需要记录和确认预清洁保留时限（＜30min）。 After the end of production/ usage, keep the dirty equipment and utensils for a specified time that listed in 8.4, and then start the cleaning operation. The recording of time should include year, month, day, hour, and minute. Among them, the high-pressure homogenization system also requires recording and verification of the hold time limit for pre-cleaning (＜30 minutes). 注：如相同的器具以及大搅拌桨使用的数量不止1个，且同步清洗，则记录最后使用器具的时间。 Note: If the same utensils and large mixing paddles are used in a quantity of more than one and they are cleaned simultaneously, record the time of the last used utensil. - 清洁过程中确认设备、器具已按清洁SOP（CIP/COP/手动清洁）准确执行，包括COP时的装载位置，并及时完成清洁记录，并且手动操作已执行最差条件。 During the cleaning process, confirm that the equipment and utensils have been accurately cleaned in accordance with the cleaning SOP (CIP/COP/manual cleaning), including the loading position during COP, and promptly complete the cleaning records, and manual operation has been performed under the worst-case conditions. - 在清洁完成后，操作人员（裸眼视力或矫正后视力应≥5.0）需在在不小于200Lux的光照（必要时可增加辅助照明）、60cm距离内对已清洁的、能够观察到的所有设备、器具物料接触表面进行目视检查，确认是否洁净、是否有可见异物（包括残留物）、是否有积水，再由QA（裸眼视力或矫正后视力应≥5.0）复核。 After the cleaning is completed, operators (with unaided or corrected vision ≥5.0) must conduct a visual inspection of all cleaned and observable equipment, utensils, and material contact surfaces at a distance of 60cm under a light intensity of no less than 200Lux (auxiliary illumination can be added if necessary). This inspection aims to verify if the surfaces of equipment and utensils are clean, free of visible foreign objects (including residues), and water accumulation. The inspection will then be reviewed by QA personnel (with unaided or corrected vision ≥5.0).

接受标准/ Acceptance Criteria

- 脏污保留时间（含预清洁保留时限）已执行（应大于规定时间）。 The dirty hold time (Including the hold time limit for pre-cleaning) has been accurately executed (should be longer than the specified time). 脏污保留时间Dirty Hold time. 生产结束/使用结束至清洁开始（期间有预清洁） Time from the end of production/usage to the start of cleaning (There is pre-cleaning during this period); 器具、大搅拌桨和原液配制工序隔膜泵DHT为72小时，高压均质系统DHT为40小时（预清洁时限为＜30min）。 The DHT for the utensils, large paddles and diaphragm pump of bulk drug substance preparation process is 72 hours, the DHT for the high-pressure homogenization system is 40 hours(The pre-cleaning time limit is ＜30 minutes.).. 生产结束/使用结束至清洁开始（期间无预清洁） Time from the end of production/usage to the start of cleaning (No pre-cleaning during the period); SEC层析工序AKTA层析系统和层析柱 DHT为24小时，CEX层析工序的 AKTA层析系统和层析柱的DHT为24小时，GST亲和层析工序的全自动层析系统含和层析柱的DHT为24小时。 The DHT for both the AKTA chromatography system and chromatographic columns used in the SEC chromatography process is 24 hours, the DHT for both the AKTA chromatography system and chromatography columns used in the CEX chromatography process is 24 hours, the DHT for both the fully automated chromatography system and chromatography column used in the GST affinity chromatography process is 24 hours.. - 清洁操作：设备、器具已按照清洁SOP（CIP/COP/手动清洁）准确清洁并完成清洁记录，并且手动操作已执行最差条件。 Cleaning operation: the equipment and utensils have been cleaned accurately according to the SOP (CIP/ COP/ Manual Cleaning) and the cleaning records have been completed, and manual operation has been performed under the worst-case conditions. - 目视检查：设备、器具表面应洁净、无可见异物（包括残留物），无积水。 Visual inspection: The surfaces of equipment and utensils should be clean, free of visible foreign objects (including residues), and without water accumulation.

结果/ Resluts

在下列表格中记录检查结果： Record the test results in the table below：

| 12.1-1大搅拌桨和器具保留时间确认记录Hold Time of Large Mixing Paddles and Utensils Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号/器具名称规格 Name and serial number of equipment, name and specification of utensils |  | 使用数量 Quantity used | 生产/使用结束时间 End time of production/ usage | 预清洁开始时间 Pre-cleaning start time | 清洁开始时间 Cleaning start time | 保留时间（应大于规定时间） (生产/使用结束至清洁开始时间) Hold time (should be longer than the stipulated time) (Time from end of production/usage to start of cleaning) |  | 是否合格 Pass or Fail |
| 产品名称 Product Name | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  |  | 批号 Lot Number |  |  |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) | V-E110-23-009 V-E110-23-010 |  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 不锈钢桶 Stainless steel barrels | 5L |  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 玻璃棒 Glass bars | N/A |  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 烧杯 Beakers | 5L |  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
|  | 2L |  |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 称量勺 Weighing spoons | N/A |  |  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

| 12.1-2设备保留时间确认记录Hold Time of Equipment Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号 Name and serial number of equipment |  | 使用数量 Quantity used | 生产/使用结束时间 End time of production/ usage | 预清洁开始时间 Pre-cleaning start time | 清洁开始时间 Cleaning start time | 保留时间（应大于规定时间） (生产/使用结束至清洁开始时间) Hold time (should be longer than the stipulated time) (Time from end of production/usage to start of cleaning) |  | 是否合格 Pass or Fail |
| 产品名称 Product Name | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  |  | 批号 Lot Number |  |  |
| 高压均质系统 High-pressure homogenization system | V-D121-22-003 | 1 |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 全自动层析系统 Fully automated chromatography system | V-E116-22-016 | 1 |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 层析柱 Chromatographic column | V-E116-22-019 | 1 |  |  |  |  |  |  |
| AKTA层析系统 （CEX层析） AKTA chromatography system (CEX chromatography) | V-E117-22-003 | 1 |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 层析柱（CEX层析） Chromatographic columns (CEX chromatography) | V-E117-22-007 |  |  |  |  |  |  |  |
|  | V-E117-22-008 |  |  |  |  |  |  |  |
| AKTA层析系统（SEC层析） AKTA chromatography system (SEC chromatography) | V-E117-23-010 | 1 |  |  |  |  |  | □ 通过Pass □ 失败Fail |
| 层析柱（SEC层析） Chromatographic columns (SEC chromatography) | V-E117-22-004 |  |  |  |  |  |  |  |
|  | V-E117-22-005 |  |  |  |  |  |  |  |
|  | V-E117-22-006 |  |  |  |  |  |  |  |
| 隔膜泵（原液配制） Diaphragm pump (Bulk drug sub-stance preparation) | V-E116-23-022 | 1 |  |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

| 12.1-3大搅拌桨和器具清洁操作及目视检查确认记录Cleaning Operation and Visual Inspection of Large Mixing Paddles and Utensils Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号/器具名称规格 Name and serial number of equipment, name and specification of utensils |  |  |  | 使用数量 Quantity used | 已执行SOP，记录和/或打印 Execute SOP, record and/or printed | 已执行最差条件 Worst-case scenario has been executed | 目视检查结果应符合要求 The visual inspection results should meet the requirements. | 确认人和复核人 Verifier and reviewer |  | 是否合格 Pass or Fail |
| 产品名称 Product Name | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  |  |  | 批号 Lot Number |  |  |  |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) |  | V-E110-23-009 V-E110-23-010 |  |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 不锈钢桶 Stainless steel barrels |  | 5L |  |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 玻璃棒 Glass bars |  | N/A |  |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 烧杯 Beakers |  | 5L |  |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
|  |  | 2L |  |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 称量勺 Weighing spoons |  | N/A |  |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 标准Criteria: 目视检查：设备、器具表面应洁净、无可见异物（包括残留物），无积水。 Visual inspection: The surfaces of equipment and utensils should be clean, free of visible foreign objects (including residues), and without water accumulation. |  |  |  |  |  |  |  |  |  |  |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

| 12.1-4设备清洁操作及目视检查确认记录Cleaning Operation and Visual Inspection of Equipment Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称规格 Name and specification of equipment |  |  | 使用数量 Quantity used | 已执行SOP，记录和/或打印 Execute SOP, record and/or printed | 已执行最差条件 Worst-case scenario has been executed | 目视检查结果应符合要求 The visual inspection results should meet the requirements. | 确认人和复核人 Verifier and reviewer | 是否合格 Pass or Fail |  |
| 产品名称 Product Name | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  |  | 批号 Lot Number |  |  |  |
| 高压均质系统 High-pressure homogenization system |  | V-D121-22-003 | 1 | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 全自动层析系统和层析柱 Fully automated chromatography system and chromatographic column |  | V-E116-22-016 V-E116-22-019 | 1 | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| AKTA层析系统（CEX层析） AKTA chromatography system (CEX chromatography) |  | V-E117-22-003 | 1 | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 层析柱（CEX层析） Chromatographic columns (CEX chromatography) |  | V-E117-22-007 |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
|  |  | V-E117-22-008 |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| AKTA层析系统（SEC层析） AKTA chromatography system (SEC chromatography) |  | V-E117-23-010 | 1 | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 层析柱（SEC层析） Chromatographic columns (SEC chromatography) |  | V-E117-22-004 |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
|  |  | V-E117-22-005 |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
|  |  | V-E117-22-006 |  | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 隔膜泵（原液配制） Diaphragm pump (Bulk drug substance preparation) |  | V-E116-23-022 | 1 | □ 是Yes □ 否No | □ 是Yes □ 否No | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |  |
| 标准Criteria: 目视检查：设备、器具表面应洁净、无可见异物（包括残留物），无积水。 Visual inspection: The surfaces of equipment and utensils should be clean, free of visible foreign objects (including residues), and without water accumulation. |  |  |  |  |  |  |  |  |  |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

  - DHT淋洗取样确认DHT Rinse Sampling Verification

目的/ Objective

- 确认脏污保留至规定时间并完成清洁操作后，淋洗取样结果符合可接受标准要求。 To verify that the rinse sampling results meet the requirements of acceptable criteria after retaining dirt for the specified time and completing the cleaning operation.

程序/ Procedures

- 淋洗取样样品Rinse Sampling Samples 设备、器具（器具的清洁操作采用制药用器具清洗干燥机清洗，取样时在设备排水口取样阀取样）在清洁结束前在相应的取样位置进行最终淋洗水取样至相应的取样器具中。检验性状、电导率、细菌内毒素、微生物限度和TOC。取样位置见附录3。 Before the cleaning of equipment and utensils is completed (The cleaning operation of the utensils are carried out using a cleaning and drying machine for pharmaceutical utensils, and sampling is conducted at the sampling valve of the equipment's discharge outlet.), the final rinse water is sampled from the corresponding sampling locations into the appropriate sampling instruments. These samples are then tested for properties, conductivity, bacterial endotoxin, microbial limit and TOC. The sampling locations are shown in the Annex 3. - 取样器具及取样量Sampling Instruments and Number of Samples 取样器具及取样量应照方案8.2执行。 Sampling instruments and sampling quantity shall be executed in accordance with protocol 8.2. - 样品编号Sample Numbering 各样品取样后应及时编号，样品编号详见附录3。 Each sample shall be numbered in time after sampling. See Annex 3 for the sample numbers. - 检测Test QC分析人员应使用经验证的分析方法或按照相关的SOP进行样品检测，确保检测结果准确。 The QC analysts should use the validated analysis method or conduct sample testing in accordance with relevant SOPs to test the samples to ensure the accuracy of the test result. - 发生任何与预期结果不符合的项目，须在备注中说明。 Any items that do not meet the expected results must be explained in the remarks.

接受标准/Acceptance Criteria

- 性状：应为无色的澄清液体；无臭。 Properties: It should be a colorless and clear liquid; odorless. - 电导率：原液配制工序的隔膜泵淋洗取样样品电导率应≤1.3μS/cm（25℃），除GST亲和层析工序、CEX层析工序和SEC层析工序的设备外的其他设备、器具淋洗取样样品电导率应≤5μS/cm（25℃）。 Conductivity: The conductivity of the rinse sampling samples from the diaphragm pump in the bulk drug substance preparation process should be ≤1.3μS/cm (at 25℃). The conductivity of the rinse sampling samples from other equipment and utensils, excluding the equipment of GST affinity chromatography process, CEX chromatography process, and SEC chromatography process, should be ≤5μS/cm (at 25℃). - TOC：高压均质系统淋洗取样样品TOC值应≤5ppm(mg/L)；器具和大搅拌桨淋洗取样样品TOC值应≤0.5ppm(mg/L)；SEC层析工序的AKTA层析系统和层析柱淋洗取样样品TOC值应≤2ppm，且不使用取样回收率对结果进行校正；其他设备淋洗取样样品TOC值应≤1ppm(mg/L)。 TOC: The TOC value of the high-pressure homogenization system of rinsing sampling samples should be ≤5ppm(mg/L); the TOC value of the utensils and large mixing paddles of rinsing sampling samples should be ≤0.5ppm(mg/L); the TOC value of the AKTA chromatographic system and chromatographic columns used in the SEC chromatography process of rinsing sampling samples should be ≤2ppm(mg/L), and without using sampling recovery factor to correct the results ; the TOC value of other equipment of rinsing sampling samples should be ≤1ppm(mg/L). - 细菌内毒素：高压均质系统和全自动层析系统、层析柱淋洗取样样品细菌内毒素应＜25EU/ml；SEC层析工序的AKTA层析系统和层析柱及SEC层析工序前的设备淋洗取样样品细菌内毒素应＜2.5EU/ml；大搅拌桨、器具和原液配制工序的隔膜泵的淋洗取样样品细菌内毒素应＜0.25EU/ml。 Bacterial endotoxin: The bacterial endotoxin of the high-pressure homogenization system and both the fully automated chromatography system and chromatographic column of rinsing sampling samples should be ＜25EU/ml; the bacterial endotoxin of both the AKTA chromatography system and chromatographic columns used in the SEC chromatography process and the equipment before the SEC chromatography process of rinsing sampling samples should be ＜2.5EU/ml; The bacterial endotoxin of the large mixing paddles, utensils and the diaphragm pump of bulk drug substance preparation process of rinsing sampling samples should be ＜0.25EU/ml. - 微生物限度：SEC层析工序前的设备淋洗取样样品微生物限度应≤100cfu/ml；大搅拌桨和器具的淋洗取样样品微生物限度应≤10cfu/100ml；SEC层析工序的AKTA层析系统和层析柱、原液配制工序的隔膜泵淋洗取样样品微生物限度应≤100cfu/100ml。 Microbial limit: The microbial limit of the equipment before the SEC chromatography process of rinsing sampling samples should be ≤100cfu/ml; the microbial limit of the large mixing paddles and utensils of rinsing sampling samples should be ≤10cfu/100ml; the microbial limit of both the AKTA chromatography system and chromatographic columns used in the SEC chromatography process, and the diaphragm pump of bulk drug substance preparation process of rinsing sampling samples should be ≤100cfu/100ml.

结果/ Results

在下列表格中记录检测结果

Record the test results in the tables below ：

| 12.2-1DHT淋洗取样确认记录DHT Rinse Sampling Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号、器具名称规格 Name and serial number of equipment, name and specification of utensils |  |  | 样品编号 Sample number | 检验项目 Test Items | 标准 Criteria |  | 检测结果 Test Results |  | 是否合格 Pass or Fail |  |
| 产品名称 Product Name |  | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  | 批号 Lot Number |  |  |  |  |
| 制药用器具清洗干燥机 Cleaning and drying machine for pharmaceutical utensils |  | V-E111-22-002 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤0.5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤10cfu/100ml |  |  |  |  |  |
| 制药用器具清洗干燥机 Cleaning and drying machine for pharmaceutical utensils |  | V-E111-22-002 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤0.5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤10cfu/100ml |  |  |  |  |  |
| 制药用器具清洗干燥机 Cleaning and drying machine for pharmaceutical utensils |  | V-E111-22-002 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤0.5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤10cfu/100ml |  |  |  |  |  |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) |  | V-E110-23-009 V-E110-23-010 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤0.5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤10cfu/100ml |  |  |  |  |  |
| 高压均质系统 High-pressure homogenization system | 高压均质机 High-pressure homogenizer | V-D121-22-003 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |  |
|  | 收获罐来液管道 The incoming solution pipeline for the harvesting tank |  |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |  |
|  | 100L收获罐 100L Harvesting tank |  |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |  |
|  | 均质机排液管道 Homogenizer discharge pipeline |  |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤5μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤5ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |  |
| 全自动层析系统 Fully automated chromatography system |  | V-E116-22-016 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | TOC | ≤1ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |  |
| 层析柱 （GST亲和层析） Chromatographic column (GST affinity chromatography) |  | V-E116-22-019 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | TOC | ≤1ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |  |
| AKTA层析系统和层析柱 （CEX层析） AKTA chromatography system and chromatographic columns (CEX chromatography) |  | V-E117-22-003 V-E117-22-007 V-E117-22-008 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | TOC | ≤1ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |  |
| AKTA层析系统和层析柱 （SEC层析） AKTA chromatography system and chromatography columns (SEC chromatography) |  | V-E117-23-010 V-E117-22-004 V-E117-22-005 V-E117-22-006 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | TOC | ≤2ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/100ml |  |  |  |  |  |
| 隔膜泵 （原液配制） Diaphragm pump (Bulk drug substance preparation) |  | V-E116-23-022 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |  |
|  |  |  |  | 电导率Conductivity | ≤1.3μS/cm（25℃） |  |  |  |  |  |
|  |  |  |  | TOC | ≤1ppm (mg/L) |  |  |  |  |  |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜0.25EU/ml |  |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/100ml |  |  |  |  |  |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

  - DHT擦拭取样TOC确认DHT Swab Sampling TOC Verification

目的/ Objective

- 确认脏污保留至规定时间并完成清洁操作后，擦拭取样TOC结果符合可接受标准要求。 To verify that the swab sampling TOC results meet the requirements of acceptable criteria after retaining dirt for the specified time and completing the cleaning operation.

程序/ Procedures

- 残留物擦拭取样操作人员具有资质。 Operators conducting residue swab sampling should be qualified. - 设备、器具擦拭取样样品Equipment and Utensils Swab Sampling Samples 每个待擦拭取样表面均使用2个拭子进行擦拭。取样时先将拭子头在注射用水中润湿，在瓶口挤出多余的超纯水，用足够的力度使拭子头能够充分接触待取样表面（拭子杆有可见轻微弯曲即可），再采用可重叠的来回往复的擦拭动作对待取样表面进行擦拭，擦拭应覆盖全部表面（见图1、2），将拭子翻转另一面，在垂直方向擦拭同一取样区域，擦拭应覆盖全部表面。擦拭完成后，在总有机碳样品瓶颈处折断拭子头，使拭子头掉入总有机碳样品瓶中。更换拭子，在垂直方向擦拭同一取样区域，擦拭应覆盖全部表面（见图1、2），同样在擦拭取样后，在总有机碳样品瓶颈处折断拭子头，使拭子头掉入总有机碳样品瓶中，最后加入40ml注射用水稀释后用于TOC检测。 Each sampling surface to be wiped shall be wiped with 2 swabs. When sampling, moisten the swab head in the ultrapure water, and squeeze out the excess water for injection at the bottle mouth, use enough force for the swab head to make complete contact with the surface to be sampled (a slight flex of the swab stick should be sufficient), and then the surface to be sampled shall be swabbed by overlapping reciprocating swabbing action, and which shall cover the whole surface (See Figure 1 and 2). Flip the swab to the other side and swab the same sampling area in a vertical direction, ensuring that the swabbing covers the entire surface. After swabbing, break the swab head at the bottle mouth of TOC sample bottles, so that the swab head falls into the TOC sample bottles. Change swab, swab the same area in a perpendicular direction (Figure 1 and 2). Similarly, after swabbing sampling, break the swab head at the bottle mouth of TOC sample bottles, so that the swab head falls into the TOC sample bottles, and finally add 40ml of ultrapure to dilute it for TOC test. - 擦拭取样位置见附录2。 Please refer to Annex 2 for the swabbing sampling location. - 取样器具及取样量Sampling Instruments and Number of Samples 取样器具及取样量应照方案8.2执行。 Sampling instruments and sampling quantity shall be executed in accordance with protocol 8.2. - 样品编号Sample Numbering 各样品取样后应及时编号，样品编号详见附录3。 Each sample shall be numbered in time after sampling. See Annex 3 for the sample numbers. - 检测Test QC分析人员应使用经验证的分析方法进行样品检测，确保检测结果准确。 The QC analysts should use the validated analysis method to test the samples to ensure the accuracy of the test results. - 发生任何与预期结果不符合的项目，须在备注中说明。 Any items that do not meet the expected results must be explained in the remarks.

接受标准/Acceptance Criteria

- TOC：高压均质系统擦拭取样样品TOC值应≤5ppm(mg/L)；GST亲和层析工序层析柱柱壳擦拭取样样品TOC值应≤1ppm(mg/L)。 TOC: The TOC value of the high-pressure homogenization system of swabbing sampling samples should be ≤5ppm(mg/L); the TOC value of the chromatographic column shell of GST affinity chromatography process of swabbing sampling samples should be ≤1ppm(mg/L).

结果/ Results

在下列表格中记录检测结果

Record the test results in the tables below ：

| 12.3-1DHT擦拭取样TOC确认记录DHT Swab Sampling TOC Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号、器具名称规格 Name and serial number of equipment, name and specification of utensils |  |  |  | 样品编号 Sample number | 检验项目 Test Items |  | 标准 Criteria |  | 检测结果 Test Results |  | 是否合格 Pass or Fail |
| 产品名称 Product Name |  | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  |  |  | 批号 Lot Number |  |  |  |
| 100L收获罐 （高压均质系统） 100L Harvesting tank (high-pressure homogenization system) | 内壁气液界面 The inner wall of the air-liquid interface |  | V-D121-22-003 |  | TOC | ≤5ppm(mg/L) |  |  |  |  | □ 通过Pass □ 失败Fail |
|  | 视镜气液界面附近 Near the air-liquid interface of the sight glass |  |  |  | TOC | ≤5ppm(mg/L) |  |  |  |  | □ 通过Pass □ 失败Fail |
|  | 搅拌桨的下表面和桨叶夹角附近 The lower surface of the stirring paddle blade and the area near the angle between the blades |  |  |  | TOC | ≤5ppm(mg/L) |  |  |  |  | □ 通过Pass □ 失败Fail |
| 层析柱 （GST亲和层析） Chromatographic column (GST affinity chromatography) | 柱壳内壁 Column shell inner wall |  | V-E116-22-019 |  | TOC | ≤1ppm(mg/L) |  |  |  |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

  - DHT擦拭取样微生物限度确认DHT Swab Sampling Microbial Limit Verification

目的/ Objective

- 确认脏污保留至规定时间并完成清洁操作后，擦拭取样微生物限度结果符合可接受标准要求。 To verify that the swabbing sampling microbial limit results meet the requirements of acceptable criteria after retaining dirt for the specified time and completing the cleaning operation.

程序/ Procedures

- 擦拭取样空白样品Swab Sampling Blank Samples 每次验证执行时，取一起灭菌后的不采样的拭子作为空白样品，且在检验前应避免被任何潜在外来微生物污染，与擦拭取样的拭子相同的处理方法，并同时、同条件下培养。 During each validation execution, a swab that has been sterilized along with others but not used for sampling will be taken as a blank sample. Prior to testing, it should be avoided from any potential contamination by foreign microorganisms. The swab should be treated in the same manner as the swab used for wiping and sampling, and cultured under the same conditions at the same time. - 设备、器具擦拭取样样品Equipment and Utensils Swab Sampling Samples 每个待擦拭取样表面均使用2个灭菌拭子进行擦拭。第一次擦拭使用的拭子是在缓冲液中润湿拭子头的拭子进行擦拭（将拭子头在缓冲液中润湿，在瓶口挤出多余的缓冲液），第二次擦拭使用干燥的拭子。擦拭过程：握住拭子尾端，以30°角与采样表面接触，平稳而缓慢的擦拭，采样面积约25cm2，左右擦拭整个过程应覆盖整个表面（见图1、2），翻转棉签，与前次擦拭移动方向垂直（见图1、2）。擦拭完成后，使用灭菌后的剪刀剪断拭子头，使拭子头掉入50ml无菌离心管中（含20ml缓冲液）。 Two sterilized swabs are used to wipe each sampling surface to be swabbed. The first swab used for swabbing is one that has its head dipped in the buffer solution (dip the swab head in the buffer solution and squeeze out the extra buffer solution at the bottle mouth). The second swab used is a dry one. The swabbing process: Holding the tail end of the swab, it is brought into contact with the sampling surface at a 30° angle and swabbed smoothly and slowly, covering an area of approximately 25cm2. The entire swabbing process should cover the entire surface (See Figure 1 and 2). The swab is then flipped, with the swabbing direction perpendicular to the previous one (See Figure 1 and 2). After swabbing is completed, use a sterilized scissors to cut off the swab head, allowing it to fall into a 50ml Sterile centrifuge tubes (include 20ml buffer solution). - 每个取样位置一般只取样1次，应避开TOC擦拭取样位置。 Only one sample is taken from each sampling location, and it is necessary to avoid the position of TOC swabbing for sampling - 取样器具及取样量Sampling Instruments and Number of Samples 取样器具及取样量应照方案8.2执行。 Sampling instruments and sampling quantity shall be executed in accordance with protocol 8.2. - 样品编号Sample Numbering 各样品取样后应及时编号，样品编号详见附录3。 Each sample shall be numbered in time after sampling. See Annex 3 for the sample numbers. - 检测Test QC分析人员应按照相关的SOP进行样品检测，确保检测结果准确。 The QC analysts should conduct sample testing in accordance with relevant SOPs to test the samples to ensure the accuracy of the test results. - 发生任何与预期结果不符合的项目，须在备注中说明。 Any items that do not meet the expected results must be explained in the remarks.

接受标准/Acceptance Criteria

- 微生物限度：灭菌拭子空白样品微生物限度应无菌生长；设备/器具擦拭取样样品微生物限度应≤25cfu/25cm2。 Microbial limit: the microbial limit of sterilized swabs blank samples should be free from any bacterial growth; The microbial limit of the equipment/ utensils of swabbing sampling samples should be ≤25cfu/25cm2.

结果/ Results

在下列表格中记录检测结果

Record the test results in the tables below ：

| 12.4-1DHT擦拭取样微生物限度确认记录DHT Swab Sampling Microbial Limit Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号、器具名称规格 Name and serial number of equipment, name and specification of utensils |  |  |  | 样品编号 Sample number | 检验项目 Test Items | 标准 Criteria | 检测结果 Test Results | 是否合格 Pass or Fail |
| 产品名称 Product Name |  | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  | 批号 Lot Number |  |  |
| 100L收获罐 （高压均质系统） 100L Harvesting tank (high-pressure homogenization system) | 内壁气液界面 The inner wall of the air-liquid interface |  | V-D121-22-003 |  | 微生物限度Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
|  | 视镜气液界面附近 Near the air-liquid interface of the sight glass |  |  |  | 微生物限度Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
|  | 搅拌桨的下表面和桨叶夹角附近 The lower surface of the stirring paddle blade and the area near the angle between the blades |  |  |  | 微生物限度Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
| 层析柱 （GST亲和层析） Chromatographic column (GST affinity chromatography) | 柱壳内壁 Column shell inner wall |  | V-E116-22-019 |  | 微生物限度Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

  - CHT目视检查确认CHT Visual Inspection Verification

目的/ Objective

- 确认清洁后或前清洁后保留时间保留至规定时间后，设备、器具物料接触表面目视符合可接受标准。 To verify that the retention time after cleaning or after pre-cleaning is maintained until the specified time, the materials contact surfaces of equipment and utensils meets the acceptable criteria visually.

程序/Procedures

- 在按8.4所列设备、器具清洁保留时间保留至规定时间后，开始目视检查。时间的记录应包括年、月、日、时和分。 After keeping the clean equipment and utensils for a specified time that listed in 8.4, and then start the visual inspection. The recording of time should include year, month, day, hour, and minute. - 操作人员（裸眼视力或矫正后视力应≥5.0）需在在不小于200Lux的光照（必要时可增加辅助照明）、60cm距离内对已清洁的、能够观察到的所有设备、器具物料接触表面进行目视检查，确认是否洁净、是否有可见异物（包括残留物）、是否有积水，再由QA（裸眼视力或矫正后视力应≥5.0）复核。 Operators (with unaided or corrected vision ≥5.0) must conduct a visual inspection of all cleaned and observable equipment, utensils, and material contact surfaces at a distance of 60cm under a light intensity of no less than 200Lux (auxiliary illumination can be added if necessary). This inspection aims to verify if the surfaces of equipment and utensils are clean, free of visible foreign objects (including residues), and water accumulation. The inspection will then be reviewed by QA personnel (with unaided or corrected vision ≥5.0).

接受标准/ Acceptance Criteria

- 清洁保留时间已准确执行（应大于规定时间）。 The clean hold time has been accurately executed (should be longer than the specified time). 清洁保留时间Clean Hold time. 清洁结束至使用前 From the end of cleaning to before use; 器具、大搅拌桨和高压均质系统CHT均为5天，原液配制工序隔膜泵CHT为24小时。 The CHT for utensils, large paddles and high-pressure homogenization system is 5 days, the CHT for diaphragm pump of bulk drug substance preparation process is 24 hours.. 前清洁结束至使用前 From the end of pre-cleaning to before use; GST亲和层析工序的全自动层析系统和层析柱的CHT为24小时，CEX层析工序的 AKTA层析系统和层析柱的CHT为24小时，SEC层析工序AKTA层析系统和层析柱的CHT为12小时。 The CHT for both the fully automated chromatography system and chromatography column used in the GST affinity chromatography process is 24 hours, the CHT for both the AKTA chromatography system and chromatography columns used in the CEX chromatography process is 24 hours, the CHT for both the AKTA chromatography system and chromatography columns used in the SEC chromatography process is 12 hours.. - 目视检查：设备、器具表面应洁净、无可见异物（包括残留物），无积水。 Visual inspection: The surfaces of equipment and utensils should be clean, free of visible foreign objects (including residues), and without water accumulation.

结果/ Resluts

在下列表格中记录检查结果： Record the test results in the table below：

| 12.5-1CHT大搅拌桨和器具保留时间确认记录Hold Time of Large Mixing Paddles and Utensils Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号/器具名称规格 Name and serial number of equipment, and name and specification of utensils |  |  | 使用数量 Quantity used | 清洁结束日期和时间 Cleaning completion date and time | 目视检查开始日期和时间 Visual inspection starts date and time | 保留时间（应大于规定时间） Hold time (should be longer than the stipulated time) | 目视检查结果应符合要求 The visual inspection results should meet the requirements. | 确认人和复核人 Verifier and reviewer | 是否合格 Pass or Fail |
| 产品名称 Product Name |  | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  |  | 批号 Lot Number |  |  |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) |  | V-E110-23-009 V-E110-23-010 |  |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 不锈钢桶 Stainless steel barrels |  | 5L |  |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 玻璃棒 Glass bars |  | N/A |  |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 烧杯 Beakers |  | 5L |  |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
|  |  | 2L |  |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 称量勺 Weighing spoons |  | N/A |  |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 标准Criteria: 目视检查：设备、器具表面应洁净、无可见异物（包括残留物），无积水。 Visual inspection: The surfaces of equipment and utensils should be clean, free of visible foreign objects (including residues), and without water accumulation. |  |  |  |  |  |  |  |  |  |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

| 12.5-2CHT设备保留时间确认记录CHT Hold Time of Equipment Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号 Name and serial number of equipment |  | 使用数量 Quantity used | 清洁结束日期和时间 Cleaning completion date and time | 目视检查开始日期和时间 Visual inspection starts date and time | 保留时间（应大于规定时间） Hold time (should be longer than the stipulated time) | 目视检查结果应符合要求 The visual inspection results should meet the requirements. | 确认人和复核人 Verifier and reviewer | 是否合格 Pass or Fail |
| 产品名称 Product Name | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  |  | 批号 Lot Number |  |  |
| 高压均质系统 High-pressure homogenization system | V-D121-22-003 | 1 |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 全自动层析系统 Fully automated chromatography system | V-E116-22-016 | 1 |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 层析柱 （GST亲和层析） Chromatographic column (GST affinity chromatography) | V-E116-22-019 | 1 |  |  |  | N/A |  |  |
| AKTA层析系统 （CEX层析） AKTA chromatography system (CEX chromatography) | V-E117-22-003 | 1 |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 层析柱（CEX层析） Chromatographic columns (CEX chromatography) | V-E117-22-007 |  |  |  |  | N/A |  |  |
|  | V-E117-22-008 |  |  |  |  |  |  |  |
| AKTA层析系统 （SEC层析） AKTA chromatography system (SEC chromatography) | V-E117-23-010 | 1 |  |  |  | □ 是Yes □ 否No |  | □ 通过Pass □ 失败Fail |
| 层析柱（SEC层析） Chromatographic columns (SEC chromatography) | V-E117-22-004 |  |  |  |  | N/A |  |  |
|  | V-E117-22-005 |  |  |  |  |  |  |  |
|  | V-E117-22-006 |  |  |  |  |  |  |  |
| 标准Criteria: 目视检查：设备、器具表面应洁净、无可见异物（包括残留物），无积水。 Visual inspection: The surfaces of equipment and utensils should be clean, free of visible foreign objects (including residues), and without water accumulation. |  |  |  |  |  |  |  |  |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

  - CHT淋洗取样确认CHT Rinse Sampling Verification

目的/ Objective

- 确认前清洁后保留时间保留至规定时间后，淋洗取样结果符合可接受标准要求。 To verify that the hold time after pre-cleaning is maintained until the specified time, the rinsing sampling results meet the acceptable criteria.

程序/ Procedures

- 在线淋洗取样样品In Place Rinse Sampling Samples 设备、器具在前清洁结束前在相应的取样位置进行最终淋洗水取样至相应的取样器具中。检验性状、电导率（如适用）、细菌内毒（如适用）和微生物限度。取样位置见附录3。 Before the pre-cleaning of equipment and utensils is completed, the final rinse water is sampled from the corresponding sampling locations into the appropriate sampling instruments. These samples are then tested for properties, conductivity (if applicable), bacterial endotoxin (if applicable), microbial limit and TOC. The sampling locations are shown in the Annex 3. - 取样器具及取样量Sampling Instruments and Number of Samples 取样器具及取样量应照方案8.2执行。 Sampling instruments and sampling quantity shall be executed in accordance with protocol 8.2. - 样品编号Sample Numbering 各样品取样后应及时编号，样品编号详见附录3。 Each sample shall be numbered in time after sampling. See Annex 3 for the sample numbers. - 检测Test QC分析人员应使用经验证的分析方法或按照相关的SOP进行样品检测，确保检测结果准确。 The QC analysts should use the validated analysis method or conduct sample testing in accordance with relevant SOPs to test the samples to ensure the accuracy of the test result. - 发生任何与预期结果不符合的项目，须在备注中说明。 Any items that do not meet the expected results must be explained in the remarks.

接受标准/Acceptance Criteria

- 性状：应为无色的澄清液体；无臭。 Properties: It should be a colorless and clear liquid; odorless. - 电导率：设备具淋洗取样样品电导率应≤5μS/cm（25℃）； Conductivity: The conductivity of equipment of rinsing samples should be ≤5μS/cm (25℃). - 细菌内毒素：SEC层析工序的AKTA层析系统和层析柱以及SEC层析工序前的设备淋洗取样样品细菌内毒素应＜2.5EU/ml。 Bacterial endotoxin: The bacterial endotoxin of both the AKTA chromatography system and chromatographic columns used in the SEC chromatography process and the equipment before the SEC chromatography process of rinsing sampling samples should be ＜2.5EU/ml. - 微生物限度：SEC层析工序前的设备淋洗取样样品微生物限度应≤100cfu/ml；SEC层析工序的AKTA层析系统和层析柱淋洗取样样品微生物限度应≤100cfu/100ml。 Microbial limit: The microbial limit of the equipment before the SEC chromatography process of rinsing sampling samples should be ≤100cfu/ml; the microbial limit of both the AKTA chromatography system and chromatographic columns used in the SEC chromatography process of rinsing sampling samples should be ≤100cfu/100ml.

结果/ Results

在下列表格中记录检测结果

Record the test results in the tables below ：

| 12.6-1CHT淋洗取样确认记录CHT Rinse Sampling Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号、器具名称规格 Name and serial number of equipment, name and specification of utensils |  |  | 样品编号 Sample number | 检验项目 Test Items | 标准 Criteria |  | 检测结果 Test Results |  | 是否合格 Pass or Fail |
| 产品名称 Product Name |  | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  | 批号 Lot Number |  |  |  |
| 全自动层析系统和层析柱 Fully automated chromatography system and chromatographic column |  | V-E116-22-016 V-E116-22-019 |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |
| AKTA层析系统和层析柱 （CEX层析） AKTA chromatography system and chromatographic columns (CEX chromatography) |  |  |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/ml |  |  |  |  |
| AKTA层析系统和层析柱 （SEC层析） AKTA chromatography system and chromatographic columns (SEC chromatography) |  |  |  | 性状Properties | 应为无色的澄清液体；无臭。 It should be a colorless and clear liquid; odorless. |  |  |  | □ 通过Pass □ 失败Fail |
|  |  |  |  | 细菌内毒素Bacterial endotoxin | ＜2.5EU/ml |  |  |  |  |
|  |  |  |  | 微生物限度Microbial limit | ≤100cfu/100ml |  |  |  |  |
| 注Note: CHT主要考虑的是CHT期间微生物增殖（必要时考虑细菌内毒素），以避免对待生产的产品造成污染。根据选择的取样方法、设备所处的工序和的设备使用方法的不同（例如前清洁带有填料的情况）制定了不同的清洁验证标准（详见《注射用重组A型肉毒毒素清洁验证主计划》（文件编号：VMP-2024-02）和《注射用重组A型肉毒毒素清洁验证风险评估报告》（文件编号：QRM-2024-01-003））。 CHT primarily focuses on microbial proliferation during the cleaning process (and bacterial endotoxin if necessary) to avoid contamination of the product to be produced. Different cleaning validation standards have been established based on the chosen sampling methods, the process stage of the equipment, and the equipment usage methods (such as the presence of filler during pre-cleaning). Detailed information can be found in the Cleaning Validation Master Plan for Recombinant Botulinum Toxin A for Injection (Doc. No.: VMP-2024-02) and the Risk Assessment Report of Cleaning Validation for Recombinant Botuli-num Toxin A for Injection (Doc. No.: QRM-2024-01-003). 除目视检查外，擦拭取样的样品检测项目主要为微生物限度，淋洗取样的样品检测项目主要是淋洗水的性状、电导率（如适用）、细菌内毒素（如适用）和微生物限度。 In addition to visual inspection, the main testing item for swab sampling is microbial limit, while the main testing items for rinse sampling are the properties of rinse water, conductivity (if applicable), bacterial endotoxin (if applicable), and microbial limit. |  |  |  |  |  |  |  |  |  |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.
备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

  - CHT擦拭取样确认CHT Swab Sampling Verification

目的/ Objective

- 确认清洁后或前清洁后保留时间保留至规定时间后，擦拭取样结果符合可接受标准要求。 To verify that the hold time after cleaning or after pre-cleaning is maintained until the specified time, the swab sampling results meet the acceptable criteria.

程序/ Procedures

- 在清洁保留时间后执行以下程序内容。 Execute the following procedures after the cleaning hold time has elapse. - 擦拭取样空白样品Contact Plates Sampling Blank Samples 每次验证执行时，取一起灭菌后的不采样的拭子作为空白样品，且在检验前应避免被任何潜在外来微生物污染，与擦拭取样的拭子相同的处理方法，并同时、同条件下培养。 During each validation execution, a swab that has been sterilized along with others but not used for sampling will be taken as a blank sample. Prior to testing, it should be avoided from any potential contamination by foreign microorganisms. The swab should be treated in the same manner as the swab used for wiping and sampling, and cultured under the same conditions at the same time. - 设备、器具擦拭取样样品Equipment and Utensils Swab Sampling Samples 每个待擦拭取样表面均使用2个灭菌拭子进行擦拭。第一次擦拭使用的拭子是在缓冲液中润湿拭子头的拭子进行擦拭（将拭子头在缓冲液中润湿，在瓶口挤出多余的缓冲液），第二次擦拭使用干燥的拭子。擦拭过程：握住拭子尾端，以30°角与采样表面接触，平稳而缓慢的擦拭，采样面积约25cm2，左右擦拭整个过程应覆盖整个表面（见图1、2），翻转棉签，与前次擦拭移动方向垂直（见图1、2）。擦拭完成后，使用灭菌后的剪刀剪断拭子头，使拭子头掉入50ml无菌离心管中（含20ml缓冲液）。 Two sterilized swabs are used to wipe each sampling surface to be swabbed. The first swab used for swabbing is one that has its head dipped in the buffer solution (dip the swab head in the buffer solution and squeeze out the extra buffer solution at the bottle mouth). The second swab used is a dry one. The swabbing process: Holding the tail end of the swab, it is brought into contact with the sampling surface at a 30° angle and swabbed smoothly and slowly, covering an area of approximately 25cm2. The entire swabbing process should cover the entire surface (See Figure 1 and 2). The swab is then flipped, with the swabbing direction perpendicular to the previous one (See Figure 1 and 2). After swabbing is completed, use a sterilized scissors to cut off the swab head, allowing it to fall into a 50ml Sterile centrifuge tubes (include 20ml buffer solution). - 取样时应避免DHT擦拭取样（TOC和微生物限度）时的表面。 During sampling, it is important to ensure that the surface is not swabbed with DHT (TOC and microbial limit). - 取样器具及取样量Sampling Instruments and Number of Samples 取样器具及取样量应照方案8.2执行。 Sampling instruments and sampling quantity shall be executed in accordance with protocol 8.2. - 样品编号Sample Numbering 各样品取样后应及时编号，样品编号详见附录3。 Each sample shall be numbered in time after sampling. See Annex 3 for the sample numbers. - 检测Test QC分析人员应按照相关的SOP进行样品检测，确保检测结果准确。 The QC analysts should conduct sample testing in accordance with relevant SOPs to test the samples to ensure the accuracy of the test results. - 发生任何与预期结果不符合的项目，须在备注中说明。 Any items that do not meet the expected results must be explained in the remarks.

接受标准/Acceptance Criteria

- 微生物限度：灭菌拭子空白样品微生物限度应无菌生长；设备/器具擦拭取样样品微生物限度应≤25cfu/25cm2。 Microbial limit: the microbial limit of sterilized swabs blank samples should be free from any bacterial growth; The microbial limit of the equipment/ utensils of swabbing sampling samples should be ≤25cfu/25cm2.

结果/ Results

在下列表格中记录检测结果

Record the test results in the tables below ：

| 12.7-1CHT擦拭取样微生物限度确认记录CHT Swab Sampling Microbial Limit Verification Record 页数Page of___ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 设备名称编号、器具名称规格 Name and serial number of equipment, name and specification of utensils |  |  |  | 样品编号 Sample number | 检验项目 Test Items | 标准 Criteria | 检测结果 Test Results | 是否合格 Pass or Fail |
| 产品名称 Product Name |  | 注射用重组A型肉毒毒素原液 Recombinant Botulinum Toxin A for Injection Bulk Drug Substance |  |  |  | 批号 Lot Number |  |  |
| 烧杯 Beakers | 杯口 The neck |  | 5L |  | 微生物限度 Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
| 不锈钢桶 Stainless steel barrels | 桶口 The neck |  | 5L |  | 微生物限度 Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) | 大搅拌桨表面 Surface of large mixing paddles |  | V-E110-23-009 V-E110-23-010 |  | 微生物限度 Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
| 100L收获罐 （高压均质系统） 100L Harvesting tank (high-pressure homogenization system) | 内壁气液界面 The inner wall of the air-liquid interface |  | V-D121-22-003 |  | 微生物限度 Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
|  | 视镜气液界面附近 Near the air-liquid interface of the sight glass |  |  |  | 微生物限度 Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |
|  | 搅拌桨的下表面和桨叶夹角附近 The lower surface of the stirring paddle blade and the area near the angle between the blades |  |  |  | 微生物限度 Microbial limit | ≤25cfu/25cm2 |  | □ 通过Pass □ 失败Fail |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

符合性Conformity                                                                                                     □是Yes     □否No

确认人/日期Verified by/Date                                                                                    复核人/日期Reviewed by /Date

- 偏差清单Deviation List

| 页数Page of___ |  |  |  |  |
| --- | --- | --- | --- | --- |
| 偏差编号 Deviation No. | 偏差日期 Deviation Date | 描述 Description | 对确认/验证影响评估结论 Assessment conclusions of the impact on qualification/validation | 纠偏措施 Corrective actions |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

确认人/日期Verified by/Date                            复核人/日期Reviewed by /Date

- 变更清单Change List

| 页数Page of__ |  |  |  |  |
| --- | --- | --- | --- | --- |
| 变更编号 Change No. | 变更日期 Change Date | 描述 Description | 对确认/验证影响评估结论 Assessment conclusions of the impact on qualification/validation | 纠偏措施 Corrective actions |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

确认人/日期Verified by/Date                            复核人/日期Reviewed by /Date

- 附件清单Attachment List

页数Page    of__

| 附件编号Attachment No. | 附件描述 Attachment Description | 对应的章节 Related Section | 总页数 Total Pages |
| --- | --- | --- | --- |

注： 如需要，本记录可复制 Note: This record may be copied as necessary.

备注Comments

确认人/日期Verified by/Date                            复核人/日期Reviewed by /Date

# 附录Annex 1：擦拭取样位置评估Swab Sampling locations Evaluation

| 设备名称编号/器具名称规格 Name and serial number of equipment, name and specification of utensils |  | 结构设计 Structural design | 风险位置 Risk position |  | 评估 Evaluation |
| --- | --- | --- | --- | --- | --- |
| 100L收获罐 （高压均质系统） 100L Harvesting tank (high-pressure homogenization system) | V-D121-22-003 | 100L收获罐：工作容积100L, 罐体圆柱形，盖子为圆盘形，上机械搅拌，单层搅拌叶（为五叶桨。罐顶配备5个接口，压力表、温度传感器、进气排气、视灯等接口，1个圆形罐口；罐盖上有两个CIP管接口，连接2个喷淋球；罐中部侧边有：温度传感器、以及条形观察窗；罐体底部装有1个底阀。 100L Harvesting tank: Working volume of 100L. the tank body is cylindrical, and the cover is disk-shaped. It is equipped with a mechanical agitator and a single-layer agitator blade (five-blade impeller). The tank top is equipped with five interfaces, including pressure gauge, temperature sensor, inlet and exhaust, observation lamp, and other interfaces, as well as one circular tank mouth. The tank cover has two CIP pipe interfaces connected to two spray balls. The middle side of the tank has a temperature sensor and a strip observation window. The bottom of the tank body is equipped with one bottom valve. | 较难清洁 Hard to Clean | 气液界面（内壁、视镜、搅拌桨桨轴）、搅拌桨的下表面和桨叶夹角附近 The air-liquid interface (including the inner wall, sight glass and stirring paddle shaft), and the lower surface of the stirring paddle blade and the area near the angle between the blades | 清洁操作采用CIP方式。 The cleaning operation employs the CIP method 内壁和视镜：由于内壁、搅拌桨桨轴均为不锈钢，清洁难易程度相当，但搅拌桨桨轴位于中心，在搅拌状态下，搅拌桨桨轴的清洁效果优于内壁，所以选择内壁相对清洁条件最差，而视镜为玻璃材质，需要确认清洁操作对玻璃材质的清洁效果，所以视镜和内壁的气液界面可作为最差条件取样位置。 Inner wall and sight glass: Since both the inner wall and the stirring paddle shaft are made of stainless steel, their cleaning difficulties are comparable. However, as the stirring paddle shaft is located at the center, its cleaning effect is superior to the inner wall when in the stirring state. Therefore, the inner wall is chosen as the one with relatively worst-case cleaning conditions. The sight glass is made of glass, and it is necessary to verify the cleaning effect of cleaning operations on glass materials. Therefore, the air-liquid interface of the sight glass and the inner wall can be used as the sampling location under the worst-case conditions. 搅拌桨的下表面和桨叶夹角附近：在淋洗过程中下表面被上表面遮挡，仅能通过清洁过程中清洁剂浸泡才能进行清洁，且夹角位置属于窄点，相对其他表面相对可清洁性较差，所以作为最差条件取样位置。 The lower surface of the stirring paddle blade and the area near the angle between the blades: During the rinsing process, the lower surface is covered by the upper surface, and can only be cleaned through soaking in cleaning agent or spraying with cleaning agent during the rinsing process, and the angle position belongs to a narrow point. Compared to other surfaces, its cleanability is relatively poorer, so it is chosen as the sampling location under the worst-case conditions. and its lower surface needs to be flushed by the reverse splashing of water sprayed from the spray ball located above the tank. As the cleaning intensity is relatively lower compared to other locations, it can be considered as a worst-case sampling location. 在较难清洁位置评估中已确定内壁、视镜、搅拌桨桨轴、搅拌桨的下表面和桨叶夹角附近（搅拌桨桨叶）中选择内壁、视镜、搅拌桨的下表面和桨叶夹角附近作为最差条件取样位置。 During the evaluation of hard to clean locations, inner wall, sight glass, the lower surface of the stirring paddle blade and the area near the angle between the blades have been identified as the worst-case sampling locations among the inner wall, sight glass, stirring paddle shaft, and the lower surface of the stirring paddle blade and the area near the angle between the blades. 罐底：罐体清洁时，除碱液清洗时为循坏外，纯化水预洗、纯化水清洗和注射用水清洗时均采用清洗液直排的方式进行清洗。注射用水清洗采用80℃注射用水进行漂洗，漂洗后干燥采用压缩空气吹扫并直排的方式干燥，不涉及最终清洗液积聚和干结在罐底的情况，所以罐底不作为最差条件取样位置，可通过淋洗取样的方式确认罐内的整体清洁情况。 Tank bottom: During the cleaning of the tank, the purified water pre-wash, purified water rinse, and water for injection rinse are all carried out using the direct discharge method of the cleaning solution, except for the alkaline solution cleaning, which is performed through circulation. The water for injection rinsing steps utilize 80℃ water for injection, followed by drying with com-pressed air blowing and direct discharge. This process does not involve the accumulation or drying of the final cleaning solution at the bottom of the tank. Therefore, the tank bottom is not considered as a worst-case sampling location. Instead, the overall cleanliness of the tank can be verified through rinse sampling. |
|  |  |  | 持续接触产品 Constant Product Contact | 搅拌桨桨叶、搅拌桨桨轴、内壁、视镜和罐底 The stirring paddles blades、 stirring paddle shaft, inner wall, sight glass and tank bottom |  |
|  |  |  | 产品积聚 Product Buildup | 罐底 Tank bottom |  |
|  |  |  | 残留物积聚 Residue Buildup | 罐底 Tank bottom |  |
|  |  |  | 窄点 Pinch Point | 搅拌桨平面和桨叶夹角 The angle between the stirring paddle plane and the blade. |  |
|  |  |  | 较难接触 Hard to Reach | 罐底 Tank bottom |  |
|  |  |  | 较难取样 Hard to Sample | 罐底 Tank bottom |  |
|  |  |  | 污染风险 Contamination Risk | 罐底 Tank bottom |  |
|  |  |  | 不规则形状 Irregular Shape | 搅拌桨桨叶 The stirring paddles blades |  |
|  |  |  | 存水 Can Retain Water | 罐底 Tank bottom |  |
|  |  |  | 气液界面 The air-liquid interface | 气液界面（内壁、视镜、搅拌桨桨轴） The air-liquid interface (including the inner wall, stirring paddle shaft and sight glass) |  |
|  |  |  | 遮挡位置 Occluded Location | 搅拌桨桨叶下表面 The lower surface of the stirring paddles blades |  |
|  |  |  | 最后清洁位置 Last Locations to Lose Residue | 罐底 Tank bottom |  |
| 层析柱（GST亲和层析） Chromatographic column (GST affinity chromatography) | V-E116-22-019 | 中空圆桶状结构，顶部和底部有尼龙筛网，中间设置有尼龙分配器，层析柱上下两端以及上柱头为316L不锈钢，侧壁为玻璃材质)，生产时装填一次性填料。工作体积6.28L。 The chromatography column has a hollow cylindrical structure with nylon screens at the top and bottom, and a nylon dispenser in the middle. The upper and lower ends of the column, as well as the upper column head, are made of 316L stainless steel, while the sidewall is made of glass. It is filled with disposable packing material during production, and its working volume is 6.28L. | 较难清洁 Hard to Clean | 层析柱下端 The lower end of the column | 层析柱下端：设备结构简单，无显著清洁死角，但由于最终清洗阶段需要组合淋洗，且由于层析柱高度以及直径影响无法进行擦拭，所以采用淋洗取样。 The lower end of the column: The equipment has a simple structure with no significant cleaning dead zones. However, due to the need for combined rinsing during the final cleaning stage and the inability to swab due to the height and diameter of the chromatography column, rinse sampling is adopted. 柱壳内壁：最终淋洗阶段为由上至下淋洗，上柱头优先进行淋洗，所以选择内壁作为擦拭取样的代表性位置。 Column shell inner wall: The final rinsing stage is rinsing from top to bottom, with the upper head taking priority. Therefore, the inner wall is chosen as the representative position for swabbing sampling. |
|  |  |  | 持续接触产品 Constant Product Contact | 层析柱上下两端、上柱头、内壁 The upper and lower ends of the column, the upper column head and the inner wall |  |
|  |  |  | 产品积聚 Product Buildup | 层析柱下端 The lower end of the column |  |
|  |  |  | 残留物积聚 Residue Buildup | 层析柱下端 The lower end of the column |  |
|  |  |  | 窄点 Pinch Point | N/A |  |
|  |  |  | 较难接触 Hard to Reach | N/A |  |
|  |  |  | 较难取样 Hard to Sample | N/A |  |
|  |  |  | 污染风险 Contamination Risk | 层析柱下端 The lower end of the column |  |
|  |  |  | 不规则形状 Irregular Shape | N/A |  |
|  |  |  | 存水 Can Retain Water | 层析柱下端 The lower end of the column |  |
|  |  |  | 气液界面 The air-liquid interface | N/A |  |
|  |  |  | 遮挡位置 Occluded Location | N/A |  |
|  |  |  | 最后清洁位置 Last Locations to Lose Residue | 层析柱下端 The lower end of the column |  |
| 综上，100L收获罐（高压均质系统）取样位置包括：内壁气液界面、视镜气液界面附近，搅拌桨的下表面和桨叶夹角附近；层析柱柱壳（GST亲和层析工序）取样位置为柱壳内壁。 In summary, the sampling locations for the 100L harvesting tank (high-pressure homogenization system) include: inner wall of the air-liquid interface, near the air-liquid interface of the sight glass, the lower surface of the stirring paddle blade and the area near the angle between the blades; the sampling location for the chromatographic column shell (GST affinity chromatography process) is the column shell inner wall. |  |  |  |  |  |

# 附录Annex 2：擦拭取样位置图例Illustration of Swabbing Sampling locations

| 设备名称编号 Name and serial number of equipment |  | 取样位置图例 Illustration of sampling locations |  |
| --- | --- | --- | --- |
| 100L收获罐 （高压均质系统） 100L Harvesting tank (high-pressure homogenization system) | V-D121-22-003 | 内壁气液界面 The inner wall of the air-liquid interface |  |
|  |  | 视镜气液界面附近 Near the air-liquid interface of the sight glass |  |
|  |  | 搅拌桨的下表面和桨叶夹角附近 The lower surface of the stirring paddle blade and the area near the angle between the blades |  |
| 层析柱（GST亲和层析） Chromatographic column (GST affinity chromatography) | V-E116-22-019 | 柱壳内壁 Column shell inner wall |  |

# 附录Annex 3：样品编号表Sample Number Table

| 器具名称规格/设备名称编号 Name and specification of utensils, name and serial number of equipment |  |  | 取样位置 Sampling Location | 取样阶段 Sampling Stage | 样品编号 Sample Number | 取样方法 Sampling method |
| --- | --- | --- | --- | --- | --- | --- |
| 制药用器具清洗干燥机 Cleaning and drying machine for pharmaceutical utensils |  | V-E111-22-002 | 设备排水口取样阀 Sampling valve at the equipment discharge outlet | DHT | 1C1D1\2\3…… 2C1D1\2\3…… 3C1D1\2\3…… | 淋洗取样 Rinse sampling |
| 烧杯 Beakers |  | 5L | 杯口 The neck | CHT | 1C2C1 2C2C1 3C2C1 | 擦拭取样 Swab sampling |
| 不锈钢桶 Stainless steel barrels |  | 5L | 桶口 The neck | CHT | 1C3C1 2C3C1 3C3C1 | 擦拭取样 Swab sampling |
| 上搅拌控制系统 （大搅拌桨） Upper mixing control systems (Large mixing paddles) |  | V-E110-23-009 V-E110-23-010 | 最终淋洗时收集淋洗水 Collect rinsing water during the final rinsing process | DHT | 1C4D1 2C4D1 3C4D1 | 淋洗取样 Rinse sampling |
|  |  |  | 大搅拌桨表面 Surface of large mixing paddles | CHT | 1C4C1 2C4C1 3C4C1 | 擦拭取样 Swab sampling |
| 高压均质系统 High-pressure homogenization system | 高压均质机 High-pressure homogenizer | V-D121-22-003 | LS0301接口 LS0301 interface | DHT | 1C5D1 2C5D1 3C5D1 | 淋洗取样 Rinse sampling |
|  | 收获罐来液管道 The incoming solution pipeline for the harvesting tank |  | CIP站排水口取样阀 Sampling valve at CIP station outlet | DHT | 1C6D1 2C6D1 3C6D1 | 淋洗取样 Rinse sampling |
|  | 100L收获罐 100L Harvesting tank |  | CIP站排水口取样阀 Sampling valve at CIP station outlet | DHT | 1C7D1 2C7D1 3C7D1 | 淋洗取样 Rinse sampling |
|  |  |  | 内壁气液界面 The inner wall of the air-liquid interface | DHT | 1C7D2 2C7D2 3C7D2 | 擦拭取样 Swab sampling |
|  |  |  |  | CHT | 1C7C1 2C7C1 3C7C1 | 擦拭取样 Swab sampling |
|  |  |  | 视镜气液界面附近 Near the air-liquid interface of the sight glass | DHT | 1C7D3 2C7D3 3C7D3 | 擦拭取样 Swab sampling |
|  |  |  |  | CHT | 1C7C2 2C7C2 3C7C2 | 擦拭取样 Swab sampling |
|  |  |  | 搅拌桨的下表面和桨叶夹角附近 The lower surface of the stirring paddle blade and the area near the angle between the blades | DHT | 1C7D4 2C7D4 3C7D4 | 擦拭取样 Swab sampling |
|  |  |  |  | CHT | 1C7C3 2C7C3 3C7C3 | 擦拭取样 Swab sampling |
|  | 均质机排液管道 Homogenizer discharge pipeline |  | CIP站排水口取样阀 Sampling valve at CIP station outlet | DHT | 1C8D1 2C8D1 3C8D1 | 淋洗取样 Rinse sampling |
| 全自动层析系统和层析柱 Fully automated chromatography system and chromatographic column |  | V-E116-22-016 V-E116-22-019 | 全自动层析系统排水口 Fully automated chromatography system discharge outlet | DHT | 1C9D1 2C9D1 3C9D1 | 淋洗取样 Rinse sampling |
|  |  |  | 最终淋洗层析柱时收集淋洗水 Collect rinsing water during the final rinsing process of chromatographic column | DHT | 1C9D2 2C9D2 3C9D2 | 淋洗取样 Rinse sampling |
|  |  |  | 柱壳内壁 Column shell inner wall | DHT | 1C9D3 2C9D3 3C9D3 | 擦拭取样 Swab sampling |
|  |  |  | 层析柱排水口（全自动层析系统和层析柱合并取样） Chromatographic column discharge outlet (combined sampling for fully automated chromatography system and chromatography column) | CHT | 1C9C1 2C9C1 3C9C1 | 淋洗取样 Rinse sampling |
| AKTA层析系统和层析柱 （CEX层析） AKTA chromatography system and chromatographic columns (CEX chromatography) |  | V-E117-22-003 V-E117-22-007 V-E117-22-008 | 层析柱排水口（AKTA层析系统和层析柱合并取样） Chromatographic column discharge outlet (combined sampling for AKTA chromatography system and chromatography column) | DHT | 1C10D1 2C10D1 3C10D1 | 淋洗取样 Rinse sampling |
|  |  |  |  | CHT | 1C10C1 2C10C1 3C10C1 | 淋洗取样 Rinse sampling |
| AKTA层析系统和层析柱 （SEC层析） AKTA chromatography system and chromatographic columns (SEC chromatography) |  | V-E117-23-010 V-E117-22-004 V-E117-22-005 V-E117-22-006 | 层析柱排水口（AKTA层析系统和层析柱合并取样） Chromatographic column discharge outlet (combined sampling for AKTA chromatography system and chromatography column) | DHT | 1C11D1 2C11D1 3C11D1 | 淋洗取样 Rinse sampling |
|  |  |  |  | CHT | 1C11C1 2C11C1 3C11C1 | 淋洗取样 Rinse sampling |
| 隔膜泵（原液配制） Diaphragm pump (Bulk drug substance preparation) |  | V-E116-23-022 | 设备排水口 Equipment discharge outlet | DHT | 1C12D1 2C12D1 3C12D1 | 淋洗取样 Rinse sampling |